Standard Building Code, the construction will be “Type V Construction” unprotected, unsprinkled.  Occupancy under SBC is Group A-2.

Occupant load is calculated:

Gym 



15 net 
 3852 sf 
= 257

Kitchen, Restrooms 

15 net
 80 sf 

= 5

Upstairs classroom

20 net
1300sf

= 65

Elevated platform stage

15 net   310 sf 

= 21

Classroom each side of stage
20 net   170 sf = 8 x 2 
= 16

Total Occupancy 




364 persons

