STANDARD
7 Apr 1999
06410
UPDATES  -

7 Apr 99
-
Revised model number of pulpit lift to model with built-in relays.

18 Feb 99
-
Updated phone number for The Pulpitman

3 Dec 98
-
Corrected city and phone number for Julius Blum

10 Nov 98
-
Moved under-Platform doors to new Section 06414.



SECTION  06410


CUSTOM CASEWORK
PART 1 GENERAL
Edit Article ‘1.1' as necessary to give an accurate, but brief description of the work of this Section and accurate listing of related work, especially if the scope of the work is not shown graphically on the Drawings.

 1.1
SUMMARY
A.
Products Supplied But Not Installed Under This Section

1.
Custom casework

B.
Related Sections

1.
Division 15  -  Plumbing
15410
 1.2
SUBMITTALS
Delete Paragraph ‘A’ if adjustable pulpit mechanism is provided under Section 06412.

A.
Product Data  -  Manufacturer's literature or cut sheets for hardware.  

B.
Shop Drawings
Shop Drawings
1.
Confirm compliance with Contract Document requirements as to configuration and dimensions of custom casework.

2.
Include plan and elevation views, materials used, standing and running trim profiles, assembly methods, joint details, fastening methods, accessories, and hardware.

Consider adding this submittal requirement as Paragraph ‘C’ if you are unfamiliar with current AWI requirements or if you do not have a current AWI manual in your office.

C.
Quality Assurance / Control

1.
Copy of AWI manual with shop drawing submission.

If desired, on larger Projects, you may include this requirement for submittal of a representative sample of the casework.  Do not use Article ‘1.3' on Projects without a significant amount of casework.

 1.3
QUALITY ASSURANCE
A.
Mock-Up  -  Prior to fabrication of complete casework package, submit section or sections of cabinetry containing typical drawer, shelving, cabinet door panel, and hardware.  Mock-ups may be installed in Project after approval.

PART 2 PRODUCTS
 2.1
MATERIALS
A.
Wood Materials

Generally, no other hardwood or veneer cut should be used.  It is also preferred that wood be clear finished rather than painted.

1.
Semi-exposed And Concealed   -  Species as acceptable for AWI custom grade.

B.
Panel Product

1.
Cores  -

a.
Cabinet Doors  -  Medium density fiberboard (MDF) with minimum density of 48 lbs per cu ft.

b.
All Other  -  Industrial grade particle board with minimum density of 45 lbs per cu ft.

2.
Facings  -

a.
Melamine or Kortron.

3.
Edgings  -

a.
Cabinet Doors And Drawer Fronts Higher Than 8 Inches  -  match doors.

b.
Shelves And Exposed Panel Product Edges   -  Hot-glued, 2 mm thick minimum, PVC edge-banding.  Wood-grained, except color matching Melamine or Kortron surface at shelf edges.

4.
Glues used in manufacture and fabrication of panel products shall be Type I or II.

5.
Moisture content shall be same as specified for lumber.

 2.2
COMPONENTS
Edit Paragraph ‘A’ to fit Project requirements.

A.
Casework Doors

1.
Doors under 1-3/8 inch thick  -  Panel Product.

B.
Cabinet And Drawer Hardware

1.
Cabinet And Drawer Pulls  -

a.
US26D steel-plated, brass / bronze core bow handles, 4 inches long minimum.

b.
Quality Standard  -
Approved Products
1)
4484 by Stanley, New Britain, CT  (800) 622-4393 www.stanleyworks.com
2)
Equal as approved by Architect before installation.  See Section 01600.

2.
Cabinet And Drawer Locks  -

a.
Pin tumbler type suitable for location.  Key individually by cabinet grouping shown on signage schedule.  Stamp keys with Room number and cabinet designation.

b.
Key cabinets and drawers within an Office alike.

c.
Acceptable Manufacturers  -
Approved Products
1)
Chicago Lock Co, Pleasant Prairie, WI  (800) 445-3204  

2)
National Cabinet Lock, Mauldin, SC  (864) 297-6655 www.nclnet.com
3)
Equal as approved by Architect before installation.  See Section 01600.

3.
Cabinet Adjustable Shelf Supports  -

a.
32mm System  -  Casework Fabricator’s standard.

b.
Standard System  -

1)
Quality Standards  -
Approved Products
a)
255 and 256 by Knape & Vogt, Grand Rapids, MI  (800) 253-1561  www.kv.com
b)
Equal as approved by Architect before installation.  See Section 01600.

Edit paragraph ‘4' by retaining or deleting paragraphs ‘b’ and ‘c’ to correspond with the sizes of cabinetry doors used on the Project.

4.
Cabinet Hinges  -

a.
European style, self-closing

b.
Doors 4 Feet High or Less  -

1)
120 degree opening minimum.

2)
Two hinges

3)
Quality Standard  -  C2R9A99 by Salice

4)
Approved Manufacturers  -
Approved Products
a)
Julius Blum Inc, Stanley NC  (800) 438-6788 www.juliusblum.com
b)
Grass America Inc, Kernersville, NC  (800) 334-3512 www.grassusa.com
c)
Knape & Vogt, Grand Rapids, MI  (800) 253-1561 www.kv.com
d)
Mepla-Alfit, High Point, NC  (800) 456-3752  www.mepla-alfit.com
e)
Soliche, Charlotte, NC  (800) 222-9652  

c.
Doors More Than 4 Feet High  -

1)
170 degree opening minimum.

2)
Four hinges minimum.

3)
Quality Standard  -  C2RFA99 by Salice

4)
Approved Manufacturers  -
Approved Products
a)
Julius Blum Inc, Stanley NC  (800) 438-6788 www.juliusblum.com
b)
Grass America Inc, Kernerville, NC  (800) 334-3512 www.grassusa.com
c)
Knape & Vogt, Grand Rapids, MI  (800) 253-1561 www.kv.com
d)
Mepla Inc, High Point, NC  (800) 456-3752  www.mepla-alfit.com
e)
Soliche, Charlotte, NC  (800) 222-9652

5.
Cabinet Inactive Leaf Catches  -

a.
Quality Standards  -
Approved Products
1)
Full-Height Doors  -  Two Surface Bolts No 043‑2 inch by H B Ives, Wallingford, CT  (203) 294-4837  

2)
All Others  -  Elbow Catch No 2 by H B Ives, Wallingford, CT  (203) 294-4837

3)
Equal as approved by Architect before installation.  See Section 01600.

6.
Drawer Guides  -

a.
Standard Drawers  -

1)
Full extension, steel ball bearings, 100 lb load rating.

2)
Approved Models And Manufacturers  -

a)
Series 3832 by Accuride

b)
Series KV8400 by Knape & Vogt

Add paragraph ‘c’ if lateral drawers are shown.

c.
Lateral Files -

1)
Full extension, steel ball bearings, 150 lb load rating.

2)
Approved Models And Manufacturers  -

Lateral Files 30 inches wide and under

a)
Series 4034 by Accuride

b)
Series KV8505 by Knape & Vogt

Lateral Files over 30 inches wide

a)
Series 3640 by Accuride

b)
Series KV8520 by Knape & Vogt

Other types of drawer guides and drawer guides by other manufacturers are not acceptable at the present time.

bb.b.
Approved Manufacturers  -
Approved Products
1)
Accuride, Santa Fe Springs, CA  (562) 903-0200 www.accuride.com
2)
Knape & Vogt, Grand Rapids, MI  (800) 253-1561 www.kv.com
If Material Center gate is numbered and appears on the Door Schedule, delete Article ‘C’.

If pulpit is provided by Owner under Section 06412, the adjustable pulpit mech​anism is included in that Section and Paragraph ‘D’ should be deleted.

 2.3
FABRICATION
A.
Construction

1.
Cabinet Body  -

a.
Use AWI Flush Overlay construction on cabinet bodies.

b.
If used, install Standard System adjustable shelf supports recessed.

2.
Drawers  -

a.
Fabricate with separate, screw-attached drawer front.

b.
Joints shall be dowel and pressure glued, or lock shoulder, glued, and pin nailed.

c.
Set bottoms into sides, backs, and subfront with 1/4 inch deep groove with 3/8 inch minimum standing shoulder.

d.
Every drawer shall have specified drawer guides and pull installed.  Install drawer guides with ‘Euroscrews’, and pulls with through-bolts passing through both front and sub-front.

3.
Cabinet Doors  -

a.
Hinges  -  Install hinges using plastic insertion dowels for hinges and ‘Euroscrews’ for baseplates.

b.
Every cabinet door shall have specified pull installed

B.
Cabinet Component Thickness And Material

1.
Use plastic laminate facing, except on following surfaces, where Kortron or Melamine shall be used.

a.
Cabinet interiors and shelving faces behind cabinet doors in all rooms.

b. Cabinet interiors and shelving faces exposed to view.

c.
Cabinet exteriors permanently concealed.

d.
Drawer sides, backs, bottoms, and subfronts.

2.
Ends, Divisions, Bottoms, Tops  -  3/4 inch thick panel product.

3.
Rails  -  3/4 inch thick panel product.

4.
Shelves  -  Panel product, 3/4 inch thick for spans up to 30 inches and one inch thick for spans over 30 inches.  Provide Hafele or equal center supports for spans over 42 inches.

5.
Backs  -  1/4 inch thick panel product.

6.
Doors  -  3/4 inch thick panel product.

7.
Drawer Sides, Backs, And Subfronts  -  1/2 inch thick minimum panel product.

8.
Drawer Bottoms  -  1/4 inch thick panel product.

9.
Separate Drawer Front

a.
8 Inches High And Less  -  3/4 inch thick solid hardwood

b.
More Than 8 Inches High  -  3/4 inch panel product

10.
Hardboard Dividers  -  1/4 inch thick panel product

11.
Hardboard Shelves  -  1/8 inch thick hardboard, smooth both sides

C.
Install plastic grommets in cable access holes in countertops.

If the pulpit is provided by Owner under Section 06412, delete Paragraph ‘D’.

PART 3 EXECUTION  -  Not Used
Installation

END OF SECTION

557-1804 (20020)
3
Section 06410

Orlando Welfare Complex 
October 11, 2000
Custom Casework


