STANDARD
11 Feb 1999
06416

UPDATES  -

11 Feb 99
-
Changed method of selecting and specifying colors.



SECTION  06416


PLASTIC LAMINATE COUNTERTOPS
PART 1 GENERAL
 1.1
SUMMARY
A.
Products Supplied But Not Installed Under This Section

1.
Wall‑hung counters

2.
Countertops for custom casework.

B.
Related Sections

1.
Division 15  -  Plumbing
15410
1.2 REFERENCES
Delete or add references based on editing of other parts of this Section.

A.
American National Standards Institute
ANSI
1.
ANSI A161.2-1979 (R1987), 'Performance Standards for Fabricated High Pressure Decorative Laminate Countertops'

B.
American National Standards Institute / National Electrical Manufacturer's Association


ANSI
1.
ANSI / NEMA LD 3-1991, 'High Pressure Decorative Laminates'

 1.3
SUBMITTALS
A.
Product Data
Product Data
1.
Manufacturer's literature for plastic laminate

2.
Color selections.

PART 2 PRODUCTS
 2.1
MATERIALS
A.
Plastic Laminates

1.
Quality  -

It is preferred that all counter front edges and backsplashes be roll formed as specified unless the situation makes it impossible.  Make certain the cabinetry details show the configuration required where roll forming is impossible.

a.
Countertops shall have post formed front edge and backsplash, except wehre detailed otherwise, with plastic laminate meeting requirements of ANSI / NEMA LD 3  -  PF 42.  No raised lip on front edge.

b.
Other laminates shall meet requirements of ANSI / NEMA LD 3  -

1)
Vertical Applications  -  GP 28.

2)
Horizontal (other than countertops)  -  GP 38.

c.
Balancing Material  -  BK 20

d.
AWI Quality Grade  -  Premium

2.
Assemblies  -

a.
Countertops shall meet requirements of ANSI A161.2.

b.
Adhesives for other than post-formed types shall be spray grade, high heat resistant, neoprene contact adhesive.

3.
Color to be selected by architect from manufacturers full color line.  

4.
Approved Manufacturers  -
Approved Products
a.

Select one of the suggested colors to be used as the Color Quality Standard, unless another is approved for this Project by the Project Manager after submission of the required color board.  It is preferred that neutral colors and inconspicuous patterns be used.

Nevamar, Odenton, MD  (800) 638-4380  www.nevamar.com
b.
Formica, Cincinnatti, OH  (513) 786-3525 www.formica.com, all matte finish

c.
Pioneer Plastics Corp., Auburn, ME (800) 746-6483 www.pionitelaminates.com 

d.
WilsonArt, Temple, TX (800) 433-3222 www.wilsonart.com
END OF SECTION

557-1804 (20020)
2
Section 06416

Orlando Welfare Complex
October 11, 2000
Plastic Laminate Countertops

