STANDARD
1 Aug 1998
07213
UPDATES  -



SECTION  07213


RIGID INSULATION
PART 1 GENERAL
Edit Article ‘1.1' as necessary to give an accurate, but brief description of the work of this Section and accurate listing of related work, especially if the scope of the work is not shown graphically on the Drawings.

 1.1
SUMMARY
A.
Includes But Not Limited To

1.
Furnish and install on interior side of perimeter foundation walls and under floor slabs as described in Contract Documents.

2.
Furnish and install in exterior metal framed walls as described in Contract Documents.

B.
Products Supplied But Not Installed Under This Section

1.
Insulation for cavity between brick and block on exterior walls.

C.
Related Sections

1.
Division 04  -  Installation in masonry cavity
04814
2.
Division 09  -  Metal exterior wall framing
09110
 1.2
REFERENCES
Delete or add references based on editing of other parts of this Section.

A.
American Society For Testing And Materials
ASTM
1.
ASTM C 578-95, 'Standard Specification for Rigid, Cellular Polystyrene Thermal Insulation'

PART 2 PRODUCTS
 2.1
MATERIALS
A.
Insulation For Above And Below Grade Installation

1.
Meet requirements of ASTM C 578, Type IV.

Equals may be specified providing they meet the ASTM requirements specified.

2.
Acceptable Products And Manufacturers  -
Approved Products
a.
Amofoam CM by Tenneco Building Products, Smyrna, GA  (800) 227-7339 www.tennecobuildingprod.com 

b.
Foamular 250 by Owens Corning, Toledo, OH  (800) 828-7155 www.owens-corning.com
c.
Styrofoam SM by Dow Chemical, Midland, MI  (800) 441-4369 www.dow.com
d.
Equal as approved by Architect before bidding.  See Section 01600.

B.
Insulation For Above Grade Installation

1.
Meet requirements of ASTM C 578, Type X or IX.

Equals may be specified providing they meet the ASTM requirements specified.

2.
Acceptable Products And Manufacturers  -
Approved Products
a.
Foamular 150 by Owens Corning, Toledo, OH  (800) 438-7465   www.owens-corning.com
b.
Grayboard by Dow Chemical, Midland, MI  (800) 441-4369   www.dow.com
c.
Equal as approved by Architect prior to bidding.  See Section 01600.

C.
Adhesive

1.
Acceptable Products And Manufacturers  -
Approved Products
a.
QB-300 by Ohio Sealants, Mentor, OH  (800) 321-3578  www.osisealants.com
b.
Multibond Finish Construction & Repair Adhesive by Franklin International, Columbus, OH  (800) 877-4583  www.franklini.com
c.
Maxbond by H. B. Fuller Co, Minneapolis, MN  (612) 782-1755 www.hbfuller.com
d.
Sonneborn 200 by Sonneborn Building Products, Minneapolis, MN  (800) 496-6067  www.chemrex.com
e.
Equal as approved by Architect before use.  See Section 01600.

 2.2
SOURCE QUALITY CONTROL
A.
Insulation shall be manufactured to be in compliance with UBC or other applicable building codes.

PART 3 EXECUTION
 3.1
INSTALLATION
Installation
A.
Install insulation in compliance with UBC or other applicable building codes and as follows

Select the option that matches the details on the Drawings.  Generally option one is preferred and both options are very rarely used together except in areas of very severe winters.

On the Legacy, option two is used at the Rostrum and option one at the rest of the building.

OPTION ONE

1.
Remove ties and concrete protrusions that would keep insulation from fully contacting foundation wall face.

2.
Install against interior side of perimeter foundation walls extending downward from top of slab 48 inches or to top of footing, whichever is less.  Install using 3/8 inch beads of specified adhesive at 12 inches on center vertically and at each vertical and horizontal joint to completely seal insulation.

OPTION TWO

1.
Install horizontally for 48 inches under building slab continuously around building perimeter and between slab and foundation wall for full depth of slab.


END OF SECTION

20020
1
Section 07213

MeetingHouse
May 22, 2000
Rigid Insulation


