OPTIONAL
6 May 1999
07634opt.a
UPDATES  -

This Section specifies a perforated soffit system.  It is generally used where no soffit strip vent is detailed.  If this system is used, check the soffit details on the Storage Building.



SECTION  07634


METAL SOFFITS
PART 1 GENERAL
 1.1
SUMMARY
A.
Includes But Not Limited To

1.
Furnish and install metal soffit as described in Contract Documents.

 1.2
SYSTEM DESCRIPTION
A.
Performance Requirements

1.
Installed soffit system shall meet minimum required structural loading conditions when tested in accordance with Test Method No. 4 of AAMA Specification 1402-86.

2.
Face finish shall meet performance requirements of Test Method No. 6 of AAMA Specification 1402.-86.  Reverse side coating shall pass requirements of paragraphs 1.1 through 1.4 of Test Method No. 6.

 1.3
WARRANTY
Warranty
A.
Manufacturer's written 20 year guarantee for finish.

PART 2 PRODUCTS
 2.1
COMPONENTS
A.
Aluminum Soffit

1.
Panels  -

a.
0.019 inch thick minimum.

b.
'V' groove design complete with matching trim.

c.
Panels shall be interlocked full length of panel.

d.
Perforated full width of panel with holes designed so one dimension does not exceed 1/8 inch.

2.
Finish  -  Double baked enamel to meet or exceed specifications of MIL‑C‑5541 and FHA BULLETIN UM 27 with protective coating on back side.  Color as selected by Architect from Manufacturer's standard colors.

Edit the list of acceptable manufacturers to include only those with whom you are familiar and whose products are available in the Project area.  You may add any manufacturers whose products you determine meet the specification and Project requirements.

3.
Acceptable Manufacturers  -
Approved Products
a.
Alcoa Building Products, Sidney, OH  (800) 962-6973 www.alcoahomes.com
b.
Alside Inc, Cuyahoga Falls, OH  (800) 257-4335  www.alside.com
c.
Gentek (Revere) Building Products, Cleveland, OH  (800) 548-4542 www.gentekinc.com
d.
Norandex, Cleveland, OH  (216) 468-2200  www.norandex.com
e.
Peterson Aluminum Corp, Elk Grove, IL  (800) 323-1960  www.pac‑clad.com
f.
Reynolds Metals Company, Richmond, VA  (804) 281-2636 www.rmc.com
g.
System 3‑12L by Rollex, Elk Grove Village, IL  (800) 251-3300 www.rollex.com
h.
Equal as approved by Architect prior to bidding.  See Section 01600.

B.
Fastening Devices

Select one of the following types of fasteners.

1.
Standard  -  1-1/4 inch galvanized staples or as recommended by Soffit Manufacturer.

1.
High Wind Areas  - Non-corrosive screws of length and type recommended by Soffit Manufacturer

PART 3 EXECUTION
 3.1
INSTALLATION
Installation
A.
Conceal fasteners where possible.  Paint heads of exposed fasteners to match background.

B.
Isolate from dissimilar metals to prevent electrolytic action.

C.
Repair buckling or bowing due to improper installation at no cost to Owner.


END OF SECTION

557-1804 (20020)
1
Section 07634[opt.a]

Orlando Welfare Complex
October 11, 2000
Metal Soffits


