STANDARD
29 Oct 1999
07723
UPDATES  -

29 oct 99
-
Updated Western Metal’s phone number

22 Mar 99
-
Updated Tricon’s phone number

11 Feb 99
-
Added Tricon Inc

This is the current, acceptable ridge vent as shown on most current standard plans.  This style ridge vent is generally preferred on all buildings unless otherwise shown on a standard plan or unless it will not provide the code required airflow for the building.



SECTION  07723


RIDGE VENTS
PART 1 GENERAL
 1.1
SUMMARY
A.
Products Supplied But Not Installed Under This Section

1.
Ridge vents

B.
Related Sections

Edit Section number to correspond with roofing system used.

.
Section 073??  -  Installation
07312 07313a 07313b 07322opt 07322sup
 1.2
SUBMITTALS
A.
Quality Assurance / Control  -  Manufacturer's design details and published installation instructions.
Quality
PART 2 PRODUCTS
 2.1
ACCEPTABLE MANUFACTURERS
Approved Products
A.
Lo-Profile Floating Ridge Vent by Metallic Products Corp, Houston, TX  (800) 356-7746  www.mpvent.com
B.
Southam & Warburton, Pleasant Grove, UT  (801) 785-3551

C.
MBCI, Salt Lake City, UT  (801) 530-4975  

D.
Tricon Inc, Orem, UT  (888) 565-9455

E.
Western Metal Products L.C., Woods Cross, UT  (888) 298-3454

F.
Other shop fabricated or manufactured product approved by Architect before bidding.  See Section 01600.

 2.2
FABRICATION
A.
Fabricated of 26 ga steel or 0.032 in thick aluminum minimum with edges returned or hemmed.

B.
Design to include polyethylene vent core.

1.
Approved Manufacturers  -
Approved Products
a.
Cor-A-Vent  (800) 837-8368  www.cor-a-vent.com
b.
Trimline  (800) 438-2920  

c.
VentSure by Owens Corning  (800) 438-7465  www.owens-corning.com
C.
Skirts and ridge cap connected through vent core with painted 1/4 by 1-1/2 inch Type A corrosion resistant screws with neoprene washers 8 inches on center each skirt.

D.
Accessories  -  End plugs, splice plates, seal tape, straps, fasteners, and other accessories necessary for proper installation.

 2.3
FINISHES

A.
Exposed Side

1.
Polyvinyledene Fluoride (PV2) Resin-base finish (Kynar 500 or Hylar 5000) for coil coating components containing 70 percent minimum PVF2 in resin portion of formula.  Thermo cured two coat system consisting of corrosion inhibiting epoxy or acrylic latex primer and top coat factory applied over properly pretreated metal.

2.
Color as selected by Architect from Manufacturer's standard colors.

B.
Reverse Side  -  Coating shall pass requirements of paragraphs 1.1 through 1.4 of Test Method No. 6.

PART 3 EXECUTION
 3.1
INSTALLATION
Installation
A.
Install in accordance with Manufacturer's installation instructions as approved by Architect and as follows

1.
Install assembled ridge vent over shingles in bed of asphalt plastic cement with painted 1/4 by one inch Type A corrosion resistant screws with neoprene washers 8 inches on center each side of vent.

2.
Join ridge vent units with screws, neoprene washers, splice plates, and tape sealant.


END OF SECTION

20020
1
Section 07723

MeetingHouse
Project Date
Ridge Vents

