
STANDARD
19 Jun 1999
08112
UPDATES  -

19 Jun 99
-
Deleted Total Opening Systems as a VMR Partner

12 Oct 98
-
Updated VMR contact information



SECTION  08112


STANDARD STEEL DOORS
PART 1 GENERAL
 1.1
SUMMARY
A.
Products Supplied But Not Installed Under This Section

1.
Hollow metal doors

Delete paragraph ‘2' in no hollow metal fire rated doors required.

B.
Related Sections

1.
Division 06  -  Door installation
06202
PART 2 PRODUCTS
 2.1
MANUFACTURED UNITS
A.
Doors

1.
Grade 1, Model 1 as specified in Steel Door Institute Bulletin SDI 100‑91.

2.
Finish  -

a.
Use either of following systems  -

1)
Before assembly, clean metal according to Fed Spec TT‑C‑490b, then prime surfaces with one shop coat.  After assembly, apply one baked-on primary coat.

2)
Galvanize

Delete paragraph ‘3' if no hollow metal doors to the exterior of the main building specified.

3.
Insulation  -  Exterior doors at exterior of main building and interior doors to be insulated to provide U‑value of 0.10 maximum.

Delete paragraph ‘4' if no hollow metal fire doors are necessary.

 2.2
APPROVED MANUFACTURERS
A.
Any current member of Steel Door Institute.

 2.3
FABRICATION
A.
General

1.
Mortise and reinforce doors for hinges and locks.

2.
Reinforce doors for closers and other surface applied hardware.

3.
Drill and tap on job.

4.
Seams along vertical edges of door need not be filled.

5.
Do not extend hinge cut out full width of door unless fill strip is inserted, weld filled, and ground smooth so no seam appears on back face plate.

Delete paragraph ‘6’ if no double hollow metal doors specified.

6. Double doors shall have overlapping rolled steel astragal.

7. Top edge to have flush closing channels with sealant.  

Delete paragraph ‘B' if no hollow metal fire doors are necessary.

 2.4
SOURCE QUALITY CONTROL
A.
Verification of Performance  -  Label each door as conforming to above required standards.

PART 3 EXECUTION
 3.1
VMR SUPPLIERS
Approved Products
These are the only suppliers authorized to provide hollow metal, wood doors, and hardware for all LDS Projects including work by FM Groups.  They are available for consultation and document preparation during design.  If the documents are not made available on Dodge / Scan or CMD due to Project size (FM Group work, for example), the VMR Suppliers must be contacted so they can be sent the appropriate documents and information.

A.
Architectural Building Supply, Salt Lake City, UT  Russ Farley

Phone (800) 574-4369
FAX 801-484-6817

B.
Beacon Metals Inc, Salt Lake City, UT  Chad Riches

Phone (888) 823-2206
FAX 801-485-7647

C.
Girtman & Associates, Nashville, TN  Lisa Ehrie

Phone (615) 350-6000
FAX 615-350-6686
E-Mail info@girtman.com


END OF SECTION

557-1804 (20020)
1
Section 08112

Orlando Welfare Complex 
October 11, 2000
Standard Steel Doors

