
STANDARD
19 Jul 1999
08115
UPDATES  -

19 Jul 99
-
Deleted Total Opening Systems as a VMR partner.



SECTION  08115


 STANDARD STEEL FRAMES
PART 1 GENERAL
 1.1
SUMMARY
A.
Products Supplied But Not Installed Under This Section

1.
Hollow metal frames

B.
Related Sections

1.
Section 08411  -  Aluminum Entry Frames
08411
2.
Division 06  -  Installation
06202
 1.2
SUBMITTALS
A.
Quality Assurance / Control  -  SDI technical bulletion 105-92, ‘Recommended Erection Instructions for Steel Frames’
Quality
PART 2 PRODUCTS
 2.1
MANUFACTURED UNITS
A.
Frames

1.
Cold rolled furniture steel.

a.
Interior Frames  -  16 gauge

b.
Exterior Frames  -  14 gauge

2.
Provide labeled frame to match fire rating of door.

3.
Finish  -

a.
Use one of following systems  -

1)
Prime surfaces with rust inhibiting primer.

2)
Galvanize.

4.
Anchors  -

a.
16 US gauge minimum meeting UL or other code acceptable requirements for door rating involved.

Delete paragraph ‘b’ if no frames installed in masonry walls.

b.
Type of masonry wall anchors used shall not interfere with vertical reinforcing steel.

 2.2
APPROVED MANUFACTURERS
Approved Products
A.
Any current member of the Steel Door Institute.

 2.3
FABRICATION
A.
General Requirements

Welded frames are generally suitable only in new construction.  If frames are to be installed in existing construction, it may be necessary to specify knock-down frames.

1.
Frames shall be welded units.  Provide temporary spreader on each welded frame.

2.
Provide Manufacturer's gauge label for each item.

3.
Make breaks, arrises, and angles uniform, straight, and true.  Accurately fit corners.

B.
Provide mortar guards at strikes and hinges.

C.
Anchors

1.
Provide 3 jamb anchors minimum for each jamb.  On hinge side, install one anchor at each hinge location.  On strike side, install one anchor at strike level and anchors at same level as top and bottom hinges.

2.
Frames installed before walls are constructed shall be provided with extended base anchors in addition to other specified anchors.

3.
Anchor types and configurations shall meet wall conditions.

PART 3 EXECUTION

 3.1
VMR SUPPLIERS
Approved Products
These are the only suppliers authorized to provide hollow metal, wood doors, and hardware for all LDS Projects including work by PM Groups.  They are available for consultation and document preparation during design.  If the documents are not made available on Dodge/Scan due to Project size (PM Group work, for example), the VMR Suppliers must be contacted so they can be sent the appropriate documents and information.

A.
Architectural Building Supply, Salt Lake City, UT  Russ Farley

Phone (800) 574-4369
FAX 801-484-6817

B.
Beacon Metals Inc, Salt Lake City, UT  Chad Riches

Phone (888) 823-2206
FAX 801-485-7647

C.
Girtman & Associates, Nashville, TN  Lisa Ehrie

Phone (615) 350-6000
FAX 615-350-6686
E-Mail info@girtman.com


END OF SECTION

557-1804 (20020)
1
08115

Orlando Welfare Complex 
October 11, 2000
Standard Steel Frames

