STANDARD
7 Mar 2000
08411

UPDATES  -

25 May 99
-
Refined threshold requirements for differing situations.

In virtually all situations where double doors are required, pairs of single doors separated by a permanent mullion should be used.  If existing double door installations without mullions are being replaced in existing buildings, it is preferred that the replacement system be a pair of single doors with a permanent mullion, if possible.  The PanicGuard system specified may only be used if pairs of doors are required and it is not possible to install a permanent mullion between them or replacement with a single door is not possible.

With prior approval of Area Office, ONE pair of entry doors into a building may be provided with a removable mullion of type specified in this Section.



SECTION  08411


ALUMINUM-FRAMED STOREFRONTS

PART 1 GENERAL

 1.1
SUMMARY

A.
Includes But Not Limited To

1.
Furnish and install aluminum storefront entry system, including hardware, glazing, and calking, as described in Contract Documents.

B.
Related Sections

1.
Division 07  -  Quality of sealants
07920
Locking cylinders will be provided at aluminum entries under Section 08712 only if high security cylinders and electronic locking systems are not being used, generally only on addition Projects where the cylinders on the existing entry doors do not have high security cylinders and the cylinders on the new doors are to match the existing.

3.
Section 08712  -  Locking cylinders

2


2

2.
Sections Under 08810  -  Quality of glass and glazing

08810 08851sup.a 08851sup.b
 1.2
REFERENCES

Delete or add references based on editing of other parts of this Section.

A.
American Society For Testing And Materials
ASTM
1.
ASTM B 221-96, 'Standard Specification for Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles and Tubes’

 1.3
SUBMITTALS

A.
Product Data
Product Data
1.
Manufacturer's literature or cut sheets for storefront system and for each item of hardware.

2.
Color and finish selections

3.
Maintenance, adjustment, and repair instructions

4.
Parts lists

B.
Shop Drawings
Shop Drawings
1.
Show locations, sizes, etc, of hardware reinforcing.

Paragraph ‘2' is necessary only if there are extensive sidelight windows and large, storefront windows above the doors and sidelights.

2.
Show wind loads and engineering for Project conditions.

 1.4
QUALITY ASSURANCE

A.
Pre-Installation Conference  -  Schedule pre-installation conference for one week prior to scheduled installation of storefront system.
Pre-Installation
 1.5
DELIVERY, STORAGE, AND HANDLING
Delivery
A.
Deliver all parts of door, together with hardware, to Project at same time.

 1.6
WARRANTY
Warranty
A.
Closers  -  Provide Closer Manufacturer's standard warranty, 10 years minimum.

PART 2 PRODUCTS

 2.1
COMPONENTS

A.
Manually Operated Doors

1.
Aluminum  -  6063 T5 aluminum alloy, or meeting requirements of ASTM B 221, alloy GS 10A‑T5

2.
Stiles And Top Rails  -  3-1/2 inches by 1-3/4 inches by 0.125 inches thick nominal

3.
Bottom Rail  -  10 inches minimum by 1-3/4 inches by 0.125 inches thick nominal

4.
Construction  -  Manufacturer's standard.

5.
Glazing Stops  -  Snap-in type with neoprene bulb-type glazing.  Units shall be glazed from exterior side.

6.
Weatherstripping  -  Neoprene bulb-type equal to Kawneer Sealair.

7.
Finish  -

Select the desired finish option.  Option 1 is to be used on new buildings in the color white.  Do not use any other colors or finishes.

Option 1

a.
Polyvinyledene Fluoride (PVF2) Resin-base finish (Kynar or Hylar) containing 50 percent minimum PVF2 in resin portion of formula and providing a pencil hardness of 3H.  Thermo cured two coat system consisting of corrosion inhibiting epoxy or acrylic latex primer and top coat factory applied over properly pretreated metal.

b.
Color to be selected from manufacture’s standard colors.  

Options 2 and 3  -  A / R Projects to match existing storefronts to remain.  If all existing storefronts are to be replaced, consider using option ‘1' above including the color white.

a.
AA-M12C22A42 Class I Dark Bronze anodized.

Option 3

a.
AA-M12C22A31 Class II minimum Clear anodized to match existing.

8.
Approved Models  -
Approved Products
a.
350 Medium Stile by Kawneer

b.
Series 375 by Vistawall

This astragal system is approved only when replacing double door entries in existing buildings where the opening is not sufficiently wide to allow a permanent mullion and a single door cannot be used.  There is no acceptable Vistawall equivalent at this time.

a.
Latching Double Doors Without Mullion  -  PanicGuard 350 by Kawneer

B.
Frames

1.
Aluminum  -  6063‑T5 aluminum alloy or meeting requirements of ASTM B 221, alloy GS 10A‑T5)

Delete paragraph ‘2' if only single doors are necessary.

Delete paragraph ‘3' if no sidelights are included.

2

2.
Sills  -  Sidelight base to match height of door bottom rail.

3.
Sealer Tape  -  3M

4.
Fasteners  -  Aluminum or non-magnetic stainless steel.  Concealed fastenings shall be cadmium or zinc-plated steel.

5.
Finish  -  Match doors

6.
Frames shall be from same Manufacturer as doors.

7.
Approved Systems  -
Approved Products
a.
Single Glazed  -

1)
Trifab '450' by Kawneer

2)
Series 2000 1-3/4x4-1/2 by Vistawall

b.
Double Glazed  -

1)
Trifab '451' by Kawneer

2)
Series 3000 2x4-1/2 by Vistawall

C.
Glazing Characteristics

1.
Interior Vestibule Glazing  -  Clear

Select the optional paragraphs ‘a’ glazing description of the glass to be installed in the aluminum storefront system.  Edit Section 08810 to match the selection shown in this Section.  The following selections of glazing characteristic combinations are permissable where double glazing is used.  Do not use reflective glazing.

Double glazing is preferred in almost all instances.

1.
Clear / Low E  -  Standard glazing, except where tinted glazing is being used.

2.
Clear / Gray -  Used in double-glazed units in geographical areas where heat gain from direct sun is a severe problem.

3.
Obscure / Low E  -  Rooms other than Foyers.  This will be applicable only in aluminum storefront windows and doors into Chapels or other rooms other than Corridors and Foyers.

4.
Obscure / Gray  -  See paragraph 2 above.

2.
Exterior Doors And Storefront At Entries  -

a.
Clear interior pane and Clear exterior pane with Low E treatment on surface 2

a.
Clear interior pane and exterior pane with Gray tinted

a.
Obscure interior pane with pattern on surface 3 and exterior pane Gray tinted

D.
VMR Manufacturers
Approved Products
1.
Kawneer, West Valley City, UT  Paul Cannon

(801) 455-4184

FAX 801-974-9033

E-Mail  -  paulcannon@aol.com

2.
Vistawall, Terrell, TX  Larry Garbert

(800) 869-4567 ex162
FAX 972-551-6264

 2.2
STOREFRONT HARDWARE

A.
Hinging  -  Top and bottom offset, ball bearing pivots per door leaf.

Use only the door closers specified in this Section.  Do not use floor closers.  If there are local code requirements for delayed closure, add that requirement.

B.
Surface-Mounted Overhead Door Closers

1.
Provide parallel arms on closers unless door position requires otherwise.

2.
Closers shall allow for 180 degree opening and not be used as a stop.

3.
Adjust closers to provide maximum opening force as required by governing code authority.

4.
Closers shall have following features  -

a.
Adjustable sweep speed

b.
Adjustable backcheck

c.
Non-handed, non-sized

5.
Approved Models And Manufacturers  -
Approved Products
a.
Surface mounted  -

1)
4041 Series parallel arm by LCN

2)
7700 Series Parallel arm by Norton

If the PanicGuard system is being used, use this paragraph instead of paragraphs ‘4' and ‘5' above.  The ‘Paneline’ exit device is approved only for use with the PanicGuard system.

4.
Approved Manufacturer -

a.
'Paneline' by Kawneer

C.
Thresholds

Delete the option not applicable for the Proejct.  If Vestibule has some other flooring material, make certain proper threshold is selected and specified.

1.
At Vestibule with Thin-Set Paver Tile -  Similar to Pemko 253 Profile

2.
At Vestibule with Paver Tile on Setting Bed  -  Manufacturer’s standard meeting handicap accessibility requirements.

D.
Sweep Strips  -  Entrance Manufacturer's standard.

E.
Stops

It is preferred that properly installed floor stops be used.  Check the layout of sidewalks leading to exterior doors and make certain that the configuration will allow installation of floor stops out of the traffic pattern.  Where this is not possible, such as in cases of three doors at an entry, use an overhead stop.  Call out requirements for overhead stops in the REMARKS or NOTES portion of the Door Schedule.

1.
Use floor stops except where required otherwise on Door Schedule.

2.
Acceptable Models And Manufacturers  -
Approved Products
a.
Floor Mounted  -

1)
FB19X by Glynn-Johnson

2)
F9079X by Trimco / BBW

3)
Equal as approved by Architect before installation.  See Section 01600.

b.
Overhead  -

1)
DH5203 Series by Corbin Russwin

2)
90S Series by Glynn-Johnson

3)
590S Series by Sargent

4)
Equal as approved by Architect before installation.  See Section 01600.

Color equivalent to medium bronze anodized may be used on medium bronze anodized storefront doors where matching existing storefront on existing buildings

F.
Push / Pulls

1.
Kawneer CPII and CO-9, clear anodized

2.
Vistawall PA16 and PH-12, clear anodized

G.
High Security Cylinders

In conjunction with the PFFM and the local stake leaders, select one of the approved, authorized lock systems specified below.  If the Stake's buildings use a security system other than those specified or if the exterior keying systems match the interior keying systems, it is recommended that existing keying system and cylinders be replaced with one of the authorized, approved high security keying systems.  The exterior door hardware should not be keyed the same as the interior hardware.  Also, only the exterior entry doors should utilize the high security cylinders.

If in an existing building it is decided to retain or match the existing standard keying system, delete Paragraphs ‘H’ and ‘I’ and other references to a high-security system.

1.
Approved Manufacturer  -
Approved Products
a.
ASSA accepting removable core with exclusive Church keyway.

a.
Medeco accepting removable core with exclusive Church keyway.

a.
Schlage accepting removable core with exclusive Church side bit milling.

H.
Removable Cores  (by Owner.  Specification for information only)
Owner
Select the optional paragraph ‘1' to correspond to the selection made in Paragraph ‘H’.

1.
ASSA Instacores with LDS-62 profile key system.

a.
Church And Factory Authorized Distributor  -

1)
Clark Security Products, 135 West 2950 South, Salt Lake City, UT

a)
Local  -  (801) 487-3227

b)
Other  -  (800) 453-6430

c)
FAX  -  (801) 487-3254

2.
Medeco cores with Level Six HUK-IC keying system with special Church keyway.

a.
Church And Factory Authorized Distributor  -

1)
Intermountain Lock & Supply Co, 3106 South Main, Salt Lake City, UT

a)
Utah  -  (800) 662-5941

b)
Other  -  (800) 453-5386

c)
FAX  -  (801) 485-7205

3.
Schlage cores with Primus Level 4+ keying system with special Church side bit milling.

a.
Church And Factory Authorized USA Distributor  -

1)
Architectural Building Supply, P O Box 65678, Salt Lake City, UT  84165-0678 or 2965 South Main St, Salt Lake City, UT  84115

a)
(801) 486-3481

b)
FAX  -  (801) 484-6817

2.3
ACCESSORIES

A.
Sealants  -  As specified in Section 07920.

 2.4
FABRICATION

A.
Construction shall meet Manufacturer's recommendations.  Joints shall be tightly closed.

B.
Mortise in manner to give maximum hardware-door connection strength and neatness of appearance.  Adequately reinforce with backplates or rivnuts to hold pivots and closers.

PART 3 EXECUTION

 3.1
VMR INSTALLERS
Acceptable Installers
Insert the names of the installers which have been provided by the VMR Manufacturers prior to bid, or contact the VMR Manufacturers directly for the information prior to bid.  Only bids from installers listed by the VMR Manufacturers and included in the Contract Documents are acceptable.  More than one installer may be listed for each manufacturer.  Do not use installer lists from other Projects without contacting VMR Manufacturer.

A.
Kawneer

B.
Vistawall

 3.2
INSTALLATION
Installation
Delete Paragraph ‘A’ if overhead stops not used.

A.
Interface With Other Work  -  When using overhead stops, coordinate installation with door closer and other door hardware.

B.
Accurately fit thresholds.  Bed thresholds in specified sealant at contact points with floor and make watertight.

C.
Use sealer tape to prevent electrolytic action.
Delete the second sentence of Paragraph ‘D’ if no sidelights on Project.

D.
Set plumb, square, level in correct alignment, and securely anchor.  If used, line up horizontal rail in sidelight with door rail.

E.
Adjust doors for proper operation after glazing entry.  Calk joints between frames and walls, both interior and exterior.

 3.3
FIELD QUALITY CONTROL

Delete the phrase set off by commas if no double doors installed.

A.
Site Tests  -  Pull test doors to ensure security of opening.

 3.4
PROTECTION

A.
Protect surface from damage until Substantial Completion Inspection.  Repair or replace damaged materials at no cost to Owner.

 3.5
HARDWARE GROUP SCHEDULE
Aluminum exterior doors from classrooms, Chapels, Serving Areas, etc, should be Exit Only and not capable of being dogged.  Exterior doors from Foyers and Corridors which are not fitted with cylinders should be capable of being dogged.  It is only necessary to provide one of a series of doors into the same Foyer or Corridor with a cylinder.  On most Projects, the Area Office will want a restricted number of doors to be provided with cylinders, normally two or three depending on location of Offices in the building, configuration of parking area, and type of building.  The other entry doors should have only exit devices which are capable of being dogged down.

These hardware groups apply only to aluminum storefront type door systems.  Other types exterior doors will be included in hardware groups in Section 08701.  Edit these groups to show the hardware options being used.

A.
Double Doors

Double entry doors to be fitted with exterior locking cylinder on one leaf, other leaf to be dogged. 

Group ST1  -  Door No' 401A, 500A, 117B 



General:  



1 each – Threshold

1 set – weatherstrip

Inactive Leaf:  

1 set – Pivots 

1 each – Push 

1 each – Pull 

1 each – Closer 

1 each – Manual Flush Bolts 

1 each – Stop 

Active Leaf:  

1 set – Pivots 

1 each – Push 

1 each – Pull 

1 each – Closer 

1 each – Stop 

1 each – Cylinder 

END OF SECTION

Legacy LE98-346

557-1804 (20020)
2
Section 08411

Orlando Welfare Complex
October 11, 2000
Aluminum-Framed Storefronts


