STANDARD
1 Aug 1998
09822

UPDATES -



SECTION  09822


ACOUSTICAL SEALANTS

PART 1 GENERAL

 1.1
SUMMARY

A.
Includes But Not Limited To

1.
Quality of sealants to be used at perimeters of and penetrations through acoustically insulated walls and associated ceilings.

 1.2
SUBMITTALS

A.
Product Data  -  Manufacturer's literature and installation recommendations.
Product Data
B.
Quality Assurance / Control  -  Certificate from Manufacturer indicating date of manufacture.
Quality
 1.3
DELIVERY, STORAGE, AND HANDLING
Delivery
A.
Handle to prevent inclusion of foreign matter, damage by water, or breakage.

B.
Deliver and keep in original containers until ready for use.

C.
Do not use damaged or deteriorated materials.

D.
Store in a cool place, but never under 40 deg F.

PART 2 PRODUCTS

 2.1
MATERIALS

A.
Sealants

1.
Sealants provided shall meet Manufacturer's shelf-life requirements.

2.
Approved Products And Manufacturers  -
Approved Products
a.
Sound Caulking by Ohio Sealants Inc, Mentor, OH  (800) 321-3578 osisealants.com
b.
Acoustical Sealant by Tremco, Beachwood, OH  (800) 321-7906 www.tremcosealants
c.
Acoustical Sealant by U S Gypsum, Chicago, IL  (800) 964-4874 www.usg.com
B.
Backing  -  Flexible closed cell polyurethane or polyolefin rod or bond breaker tape as recommended by Sealant Manufacturer for joints being sealed.

PART 3 EXECUTION

 3.1
PREPARATION

A.
Surfaces shall be clean, dry, and free of dust, oil, grease, dew, or frost.

B.
Joint Backing

1.
Rod for open joints shall be at least 1-1/2 times width of open joint and of thickness to give solid backing.  Backing shall fill up joint so depth of sealant bite is no more than 3/8 inch deep.

2.
Apply bond-breaker tape in shallow joints as recommended by Sealant Manufacturer.

 3.2
APPLICATION
Installation
A.
Install at perimeter joints and mechanical and electrical penetrations in sound insulated rooms.  Apply sealant with hand-calking gun with nozzle of proper size to fit joints.  Use sufficient pressure to insure full contact to both sides of joint to full depth of joint.

B.
Tool joints immediately after application of sealant if required to achieve full bedding to substrate or to achieve smooth sealant surface.

C.
Depth of sealant bite shall be 1/4 inch minimum and 1/2 inch maximum, but never more than one half or less than one fourth joint width.

D.
Do not apply calking at temperatures below 40 deg F.

 3.3
CLEANING

A.
Clean adjacent materials which have been soiled immediately (before setting) as recommended by Manufacturer.


END OF SECTION

557-1804 (20020)
1
Section 09822

Orlando Welfare Complex
October 11, 2000
Acoustical Sealants

