STANDARD
7 Mar 2000
09901

UPDATES  -

20 Jul 99
-
Changed pre-installation meeting requirements.



SECTION  09901


GENERAL PAINTING AND COATING REQUIREMENTS

PART 1 GENERAL

 1.1
SUMMARY

A.
Includes But Not Limited To

1.
General procedures and requirements for painting and coating.

B.
Related Sections

1.
Division 02  -  Pavement Marking
02763
2.
Division 05  -  Shop priming of steel and iron
05081
3.
Division 15  -  Mechanical identification, refrigerant line insulation, and duct interiors.
15075 15087 15812
 1.2
SUBMITTALS

A.
Product Data
Product Data
1.
Schedule with section corresponding to each specification Section covering a painting system for a substrate, arranged in same order as in Project Manual.  Each entry in schedule to include following information.

a.
Indicate each item or surface upon which each system is to be applied, colors to be used, and quantity of each component of paint system anticipated for Project.

b.
Manufacturer's cut sheets for each component of system indicating ingredients and percentages by weight and by volume, environmental restrictions for application, and film thicknesses and spread rates.

c.
Manufacturer’s substrate preparation instructions for each substrate and surface condition occurring on Project.

2.
Provide two copies of Product Data submission, one copy to be kept on Project site and second copy to be included in Operations And Maintenance Manual.

B.
Samples  -  Provide paint card for each color and for each paint system.  Card to show each coat of system separately as well as total system.
Samples
 1.3
QUALITY ASSURANCE

A.
Field Samples

1.
Before application of any paint system, if required by Architect, meet on Project site with Architect, Owner's representative, and Manufacturer's representative.  Architect may select one surface for application of each paint system specified.

2.
Apply paint systems to surfaces indicated by Architect following procedures outlined in Contract Documents and Product Data submission specified above.

3.
After approval of samples, proceed with application of paint system throughout Project.

B.
Pre-Installation Conferences
Pre-installation
1.
Participate in pre-installation conference specified in Section 09250 to review finish requirements of gypsum wallboard.

2.
Schedule painting pre-installation conference after delivery of paint but prior to or at same time as application of field samples.

1.4
DELIVERY, STORAGE, AND HANDLING
Delivery
A.
Deliver specified products in sealed, original containers with Manufacturer’s original labels intact on each container.  Deliver amount of materials indicated on submittal as necessary to meet  Project requirements in single shipment.  Notify Architect two working days prior to delivery of paint.

B.
Store materials in single place.

C.
Keep storage area clean and rectify any damage to area at completion of work of this Section.

 1.5
PROJECT CONDITIONS

A.
Project Environmental Conditions

1.
Main​tain temperature of paint storage area at 55 deg F minimum.

2.
Perform painting operations at temperature conditions recommended by Manufacturer for each operation and for each product.

 1.6
SCHEDULING

A.
Coordinate with other trades for materials and systems that require painting prior to installation.

Delete Article ‘1.7' if no extra painting materials are required.  Extra materials should only be required on larger Projects and Projects where a storage area for extra materials will be available.

 1.7
MAINTENANCE

A.
Extra Materials  -  Provide painting materials in Manufacturer's original containers and with original labels in each color used.  Provide one quart of each finish coat and one pint of each primer and of each undercoat in each color used.  Label each can with color name, mixture instructions, date, and anticipated shelf life.

PART 2 PRODUCTS

 2.1
MATERIALS

A.
Linseed oil, shellac, turpentine, and other painting materials shall be pure, of highest quality, and bear identifying labels on containers.

B.
Tinting color shall be best grade of type recommended by Manufacturer of paint or stain used on Project.

PART 3 EXECUTION

 3.1
ACCEPTABLE APPLICATORS

A.
Applicator shall have experience in application of specified products for five years minimum and be acceptable to Architect and Manufacturer.

 3.2
PREPARATION

A.
Protection

1.
Remove rags and waste used in painting operations from building each night.  Take every precaution to avoid danger of fire.

2.
Protect other finish work and adjacent materials during painting.  Do not splatter, drip, or paint surfaces not intended to be painted.  These items will not be spelled out in detail but pay special attention to the following  -

a.
Do not paint finish copper, bronze, chromium plate, nickel, stainless steel, anodized aluminum, or monel metal except as explicitly specified.

B.
Surface Preparation

1.
Fill holes and cracks in surfaces to receive paint or stain.

2.
Surfaces to be painted shall be clean and free of loose dirt.  Clean and dust surfaces before painting or finishing.

a.
Clean metal surfaces as recommended by Paint Manufacturer before applying materials.

b.
Clean shop coats of primer or paint that have become marred, and touch up with proper type primer.

c.
Treat galvanized metal and zinc surfaces as specified and in accordance with Paint Manufacturer's directions before applying first paint coat.

3.
Do no exterior painting while surface is damp, unless recommended by Manufacturer, nor during rainy or frosty weather.  Interior surfaces shall be dry before painting.  Moisture content of materials to be painted shall be within tolerances acceptable to Paint Manufacturer.

4.
Sand woodwork smooth in direction of grain leaving no sanding marks.  Clean surfaces before proceeding with stain or first coat application.

 3.3
APPLICATION
Installation
Edit Paragraphs ‘1' through ‘4' below and the SUMMARY Articles of each painting and coating Section under 09900 to specify exactly all items and surfaces to be painted or coated.  Other document preparation options are to include such information on the Finish Schedule or other specially prepared schedule.

A.
Paint or finish complete all surfaces to be painted as described in Contract Documents, including but not limited to following items.

1.
Finish casework and wood trims specified to be installed under Section 06201.  Back prime wood elements to be installed against concrete or masonry or which may be subjected to moisture.

2.
Paint mechanical and electrical items that require field painting as determined by Architect.  These include but are not limited to  -

a.
Gas pipe from gas meter into building

b.
Metal protective structures for refrigerant lines

c.
Mechanical flues and pipes penetrating roof

d.
Electrical panel and disconnect enclosures

B.
In multiple coat paint work, tint each coat with a slightly different color, but approximating shade of final coat, so it is possible to check application of specified number of coats.  Final coat shall be tinted to required color.

C.
Spread materials smoothly and evenly.  Apply coats to not less than wet and dry film thicknesses and at spreading rates for specified product as recommended by Manufacturer.

D.
On wood to receive a transparent finish, putty nail holes in wood after application of stain using natural colored type to match wood stain color.  Bring putty flush with adjoining surfaces.

E.
Touch up suction spots after application of first finish coat.

F.
Paint shall be thoroughly dry and surfaces clean before applying succeeding coats.

G.
Use fine sandpaper between coats as necessary to produce even, smooth surfaces.

H.
Make edges of paint adjoining other materials or colors clean, sharp, and without overlapping.

I.
Finished work shall be uniform, of approved color, smooth, and free from runs, sags, defective brushing, rolling, clogging, and excessive flooding.

 3.4
ADJUSTMENT

A.
After completion of painting and finishing work in any one portion of Project and before completion of total Project, repair and touch-up surfaces damaged by other trades.  Performance of this work to be at expense of trade or trades causing damage and at no additional expense to Owner.

 3.5
CLEANING

A.
Do not discard paint containers without Architect's written approval to allow count to determine if paint delivered was applied.

B.
As work proceeds and upon completion of work of any painting Section, remove paint spots from floors, walls, glass, or other surfaces and leave work clean, orderly, and in acceptable condition.  Remove debris caused by work of paint Sections from premises.

 3.6
PAINT COLOR SCHEDULE

A.
Color to be selected by Architect from manufacturers full color line.  

Select a neutral or off-white color for interior walls and ceilings rather than pastels or other colors that ‘coordinate’ with the colors of the other interior elements.  Colors for trim elements may be darker colors, but should be an unobtrusive color.  Provide name and designation of selected color(s) and name of Manufacturer.


END OF SECTION

ADVANCE \d252
B L A N K   P A G E

557-1804 (20020)
5
Section 09901

Orlando Welfare Complex
October 11, 2000
General Painting And Coating Requirements

