STANDARD
11 Feb 1999
10160

UPDATES  -

11 Feb 99
-
Changed method of selecting and specifying colors.

This Section specifies the standard metal toilet partitions to be used on all projects unless explicitly requested otherwise by the Area Office for high humidity areas.  No other types of compartments are authorized.



SECTION  10160


METAL TOILET COMPARTMENTS

PART 1 GENERAL

 1.1
SUMMARY

A.
Products Supplied But Not Installed Under This Section

1.
Metal toilet compartments

B.
Related Sections

1.
Division 06  -

Delete paragraph ‘a’ if walls are not frame construction.

a.
Blocking for compartment installation and door bumper
06110
b.
Installation
06201
2.
Section 10811  -  Commercial Toilet Accessories
10811
 1.2
REFERENCES

A.
American Society For Testing And Materials
ASTM
1.
ASTM B 86-98, 'Standard Specifications for Zinc-Alloy Die Castings'

 1.3
SUBMITTALS

A.
Product Data
Product Data
1.
Manufacturer's literature or cut sheet

2.
Color selection

PART 2 PRODUCTS

 2.1
MANUFACTURED UNITS

A.
Toilet And Miscellaneous Partitions

1.
Ceiling supported

2.
Panels  -

a.
Core type, galvanized bonderized steel sheets (minimum 0.00015 inch zinc coating), edges bound interlocked with drawn molding welded on corners.

1)
Panels, Pilasters, And Screens  -  20 gauge minimum

2)
Doors  -  22 gauge minimum

b.
Anchors  -  Non-corrosive.

3.
Pilasters  -  20 gauge minimum of same construction and finish as panels.  One per stall.

4.
Shoe  -  Stainless Steel.

5.
Hardware  -

a.
Each Door  -

1)
Gravity type hinges with double handed, nylon bottom cam, adjustable for partial door closing position, bottom hinge finished flush with door bottom.

2)
Sliding door bolt.

3)
Door strike and keeper with rubber bumper.

4)
Coat hook / door bumper.

b.
Finish  -  US 26.

c.
Meet requirements of ASTM B 86, Alloy AG 40A.

Select one of the two following finish options.  Select color from an approved Manufacturer’s standard colors and show on required color board for approval by Project Manager.

Standard finish option.

B.
Finish And Color

1.
Baked enamel finish

2.
Color to be selected by Architect from manufacturer’s full color line.  

Specify powder-coated finish option for geographical areas with problems with moisture and humidity.

Edit the list of approved manufacturers to include only those with whom you are familiar and whose products are available in the Project area.  Do not add or approve any other manufacturers or systems without written approval of Architectural & Engineering Services.

 2.2
APPROVED MANUFACTURERS
Approved Products
Include this group of manufacturers if baked enameled partitions are specified.

A.
Accurate Partitions Inc, LaGrange, IL  (708) 442-6801

B.
AAMCO - All American Metal Corp, Freeport, NY  (516) 623-0222

C.
AMPCO Products Inc, Sanger, TX  (817) 458-7401  www.ampco.com
D.
Flush-Metal Partition Corp, Maspeth, NY  (718) 784-3380  

E.
General Partitions, Erie, PA  (814) 833-1154  www.genpartitions.com
F.
Global Steel Products Corp, Deer Park, NY  (516) 586-3330 www.globalpartitions.com
G.
Hadrian Inc, Mentor, OH  (216) 974-7120  www.hadrian-inc.com
H.
Knickerbocker Partitions Corp, Freeport, NY  (516) 546-0550

I.
Metpar, Westbury, NY  (516) 333-2600  www.metpar.com
J.
Mills Company, Cleveland, OH  (440) 442-2900  www.millspartitions.com
K.
Sanymetal, Somerset, KY  (606) 678-2700  www.sanymetal.com
L.
Weis / Robart, Wayne, MI  (313) 467-8711 www.weisrobart.thomasregister.com/olc/weisrobart
Include this group of manufacturers if powder-coated partitions are specified.

A.


PART 3 EXECUTION

Delete Article ‘3.1' if walls are not frame construction.

 3.1
EXAMINATION

A. Verify that necessary blocking has been installed in framed walls for partition installation and for place where coat hook / door bumper will strike wall.

B. Verify placement of ceiling support for attachment of pilaster.  

 3.2
INSTALLATION
Installation
A.
Install pilasters rigid, plumb, and level.  Anchor pilaster to ceiling support with 3/8 inch bolts as recommended by the manufacturer.  

B.
Secure pilasters to supporting structure and level, plumb, and tighten.  Hang doors and adjust so doors are level with bottoms of pilasters when doors are in closed position.  

 3.3
ADJUSTING

A.
Lubricate hardware as recommended by Manufacturer.

B.
Set hinges on out-swinging doors to return to nearly closed position.

C.
Perform final adjustments to pilaster leveling devices, door hardware, and other operating parts of partition assembly just before Substantial Completion.

 3.4
CLEANING

A.
Remove protective masking.  Clean exposed surfaces of partitions, hardware, fittings, and accessories.

B.
Touch-up minor scratches and other finish imperfections using materials and methods recommended by Manufacturer.  Replace damaged or severely scratched materials with new materials.


END OF SECTION

ADVANCE \d252
B L A N K   P A G E

557-1804 (20020)
1
Section 10160

Orlando Welfare Complex
October 11, 2000
Metal Toilet Compartments

