SECTION 10530

HANGER ROD CANOPY

PART 1 GENERAL 

1.1 SUMMARY 

A. Includes but not limited to 

1. Furnish and install hanger rod canopy as described in contract documents.  

1.2 SUBMITTALS 

A. Product Data 

1. Manufacturer’s literature or cut sheets 

2. Color samples 

B. Samples – samples of actual canopy materials showing full range of manufacturer’s standard colors.  

PART 2 PRODUCTS 

2.1 MANUFACTURED UNITS 

A. Hanger rod canopy 

1. Roll formed aluminum interlocking deck with all extruded aluminum framing and fascia.  

2. 3003-H14 or 5005-H14 roll-formed .032 aluminum.  

3. Framing consisting of 6063-T6 extruded aluminum.  

4. All fasteners plated or stainless-steel as provided by manufacturer.  

5. Standard finish to be selected from manufacturer’s standards.  

6. Roof deck to be roll-formed interlocking self-flashing .032 aluminum of “W” profile.  

7. Projection to be 4’-0”.  

8. Fascia to be 8” high.  

PART 3 EXECUTION 

3.1 INSTALLATION 

A. Install using support columns of minimum 2 ½ x 3 tubes and not beams of minimum 5” deep with minimum wall thickness of .100.  

END OF SECTION

557-1804 (20020)
2
Section 10530 

Orlando Welfare Complex
October 11, 2000
Hanger Rod Canopy

557-1804 (20020)
1
Section 10530

Orlando Welfare Complex
October 11, 2000
Hanger Rod Canopy

