OPTIONAL
1 Aug 1998
11132opt

UPDATES  -

This type screen is authorized only for stake centers, and then only in Chapels and / or Cultural Centers.  Stake Centers are generally authorized one screen only in either the Chapel or the Cultural Center, unless shown otherwise on a standard plan.  Meetinghouses without stake suites utilize portable screens to be checked out of the Material Center and are not authorized to utilize these built-in screens.

In some cases, if a room in a meetinghouse is designated for seminary use, a projection screen will be required by the Area Office.  In this instance, the screen will be manually operated and surface mounted.



SECTION  11132


PROJECTION SCREENS

PART 1 GENERAL

 1.1
SUMMARY

A.
Products Supplied But Not Installed Under This Section

1.
Projection screen

B.
Related Sections

1.
Division 06  -  Installation
06201
 1.2
SUBMITTALS

A.
Product Data
Product Data
1.
Manufacturer's literature or cut sheet

2.
Operating instructions

B.
Quality Control  -  Manufacturer's written installation instructions
Quality
PART 2 PRODUCTS

 2.1
MANUFACTURED UNITS

A.
Projection Screen

1. Exposed, wall -mounted, manually operated screen unit.



2.
Screen Case:  Fabricated in one piece from not less than 22-gauge steel with flat back design, vinyl-covered or baked-enamel finish, and end caps with integral roller brackets and furnished with universal mounting brackets in finish matching end cape to enable attachment to wall or ceiling.  



3.
Screen:  Mildew-and flame-resistant glass fiber fabric with vinyl-coated viewing surface complying with requirements indicated below, with top edge mounted on, and securely anchored to, a 3” diameter rigid steel spring roller and bottom edge formed into a pocket holding a tubular metal slat, with ends or rod protected by plastic cape containing a screw-attached saddle and pull.  



4.
Size of Viewing Surface:  6 feet high by 6 feet wide.  



5.
Type of viewing surface:  Matte white with minimum gain characteristics complying with FS GG-S-00172D(1) for Type A screen surface.  



6.
Edge Treatment:  Black masking borders.  



7.
Approved Manufacturers – 

a. Series 500 Bredford Manufacturing Inc., Franklin Park, IL  (800) 521-9614 




www.bretford.com
b. Video C Da-Lite Screen Co., Warsaw, IN  (800) 622-3737 www.da-lite.com
c. Luma 2 Draper Shade & Screen Co., Spiceland, IN  (800) 238-7999 



www.draperinc.com
PART 3 EXECUTION  -  Not Used
Installation
END OF SECTION

557-1804 (20020)
1
Section 11132

Orlando Welfare Complex
October 11, 2000
Projection Screens


