Summary Report for Roof Damage at MECO Building # 1 Port Of New Orleans

MECO Bldg 1 is a metal-framed structure divided into several work areas located on the Industrial Canal side of New Orleans LA at the Port of New Orleans. The facility has metal wall panels and three different roof areas with two different metal roof systems. There was also a small canopy located on the north side of the building. Area A is metal R-panel roof system with an area of approximately 8240 SF. Area B has a metal R-panel roof system with an area of approximately 2380 SF., and area C is a small building located on the west side of the structure that has a corrugated metal panel roof system with an area of approximately 340 SF. The canopy is a R-panel covering with an area of approximately 348 SF.

The initial investigation reveled that area A had significant damage to the roof system due to projectile damage and wind uplifted panels. Approximately 50% of the roof was damaged. There was damage to the wind turbines, eve and gable trim, and metal ridge cap. Area B had very little damage and appears to be a newer roof. Roof area C appeared to have an eloastomeric coating applied to large parts of the roof system. The canopy sustained the loss of a few panels on the west side.

A subsequent investigation performed on 2-15-08 reveled that there have been little if any repairs to all roof areas. The gutter and downspout system has for the most part been damaged sense the storm and very little remains intact at all roof areas. The canopy located on the north side of the building has collapsed and is beyond repair. Due to damaged bay doors the inside of the structure has been exposed to the wind and elements. The vinyl backed insulation between the roof panels and the metal purling has been blown out in most areas and needs to be replaced. Replace roof and insulation.

