STATE OF LOUISIANA

STATE POLICE TROOP L HEADQUARTERS

HVAC SYSTEM REPLACEMENT

MANDEVILLE, LOUISIANA


Copyright 2002 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 01 73 29
CUTTING AND PATCHING
This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes procedural requirements for cutting and patching.

1.3 DEFINITIONS

A. Cutting:  Removal of in-place construction necessary to permit installation or performance of other Work.

B. Patching:  Fitting and repair work required to restore surfaces to original conditions after installation of other Work.

1.4 UNIT PRICES

A. The measurement for payment for Demo/Cut CMU Wall(s) will be per square foot.
B. Payment will be made at the contract price for Demo/Cut CMU Wall(s) for this scheduled item.  Payment will constitute full compensation for furnishing all materials, equipment, tools, all labor and other incidentals necessary to complete the Demo/Cut of CMU Wall(s). 
1.5 QUALITY ASSURANCE

This Article contains requirements that expand provisions contained in AIA Document A201.

A. Operational Elements:  Do not cut and patch operating elements and related components in a manner that results in reducing their capacity to perform as intended or that results in increased maintenance or decreased operational life or safety.
List below is an example only.  Revise to suit Project's operating systems.  With advice of counsel, delete below if Architect's approval is not required.  If list is deleted, delete option in paragraph above.

B. Visual Requirements:  Do not cut and patch construction in a manner that results in visual evidence of cutting and patching.  

Normally delete paragraph below.  Revise if cutting and patching are extensive or if careful coordination between several trades is necessary to avoid conflicts.

1.6 WARRANTY

Delete this Article if no warranties exist that would be affected.

A. Existing Warranties:  Remove, replace, patch, and repair materials and surfaces cut or damaged during cutting and patching operations, by methods and with materials so as not to void existing warranties.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. General:  Comply with requirements specified in other Sections.

B. In-Place Materials:  Use materials identical to in-place materials.  For exposed surfaces, use materials that visually match in-place adjacent surfaces to the fullest extent possible.

1. If identical materials are unavailable or cannot be used, use materials that, when installed, will match the visual and functional performance of in-place materials.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine surfaces to be cut and patched and conditions under which cutting and patching are to be performed.

1. Compatibility:  Before patching, verify compatibility with and suitability of substrates, including compatibility with in-place finishes or primers.

2. Proceed with installation only after unsafe or unsatisfactory conditions have been corrected.

3.2 PREPARATION

A. Temporary Support:  Provide temporary support of Work to be cut.

B. Protection:  Protect in-place construction during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of Project that might be exposed during cutting and patching operations.

C. Adjoining Areas:  Avoid interference with use of adjoining areas or interruption of free passage to adjoining areas.

Retain paragraph below where Owner continues to occupy other portions of an existing facility.

D. Existing Utility Services and Mechanical/Electrical Systems:  Where existing services/systems are required to be removed, relocated, or abandoned, bypass such services/systems before cutting to prevent interruption to occupied areas.

3.3 PERFORMANCE

A. General:  Employ skilled workers to perform cutting and patching.  Proceed with cutting and patching at the earliest feasible time, and complete without delay.

1. Cut in-place construction to provide for installation of other components or performance of other construction, and subsequently patch as required to restore surfaces to their original condition.

B. Cutting:  Cut in-place construction by sawing, drilling, breaking, chipping, grinding, and similar operations, including excavation, using methods least likely to damage elements retained or adjoining construction.  If possible, review proposed procedures with original Installer; comply with original Installer's written recommendations.

1. In general, use hand or small power tools designed for sawing and grinding, not hammering and chopping.  Cut holes and slots as small as possible, neatly to size required, and with minimum disturbance of adjacent surfaces.  Temporarily cover openings when not in use.

2. Finished Surfaces:  Cut or drill from the exposed or finished side into concealed surfaces.

3. Concrete and Masonry:  Cut using a cutting machine, such as an abrasive saw or a diamond-core drill.

4. Excavating and Backfilling:  Comply with requirements in applicable Division 31 Sections where required by cutting and patching operations.

Revise subparagraph below to suit Project.

5. Mechanical and Electrical Services:  Cut off pipe or conduit in walls or partitions to be removed.  Cap, valve, or plug and seal remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after cutting.

Retain subparagraph below if required to prevent multiple cutting and patching in the same area.  Add specific requirements for multiple contracts and special conditions requiring coordination.

6. Proceed with patching after construction operations requiring cutting are complete.

C. Patching:  Patch construction by filling, repairing, refinishing, closing up, and similar operations following performance of other Work.  Patch with durable seams that are as invisible as possible.  Provide materials and comply with installation requirements specified in other Sections.

1. Inspection:  Where feasible, test and inspect patched areas after completion to demonstrate integrity of installation.

2. Exposed Finishes:  Restore exposed finishes of patched areas and extend finish restoration into retained adjoining construction in a manner that will eliminate evidence of patching and refinishing.

a. Clean piping, conduit, and similar features before applying paint or other finishing materials.

b. Restore damaged pipe covering to its original condition.

Insert specific refinishing requirements for floors, walls, and ceilings.  Revise subparagraph and associated subparagraph below to suit Project.

D. Cleaning:  Clean areas and spaces where cutting and patching are performed.  Completely remove paint, mortar, oils, putty, and similar materials.

Insert specific installation requirements if not specified elsewhere.  Specific installation requirements are better specified in individual Sections.

END OF SECTION 01 73 29

CUTTING AND PATCHING
01 73 29 - 3

