
SECTION 16012PRIVATE 

UNDERGROUND ELECTRICAL CONSTRUCTION
PART 1
GENERAL

1.01
SUMMARY


A.
Provide all excavation and backfill required for the installation of electrical work.


B.
Contractor shall be responsible for the proper layout and the establishment of all lines and levels required for the execution of his work.  Coordination of trades to avoid interference, meet clearances, etc. is the Contractor's responsibility.


C.
No excavation and backfill shall be done within drip line of trees not shown to be removed on architectural drawings.  No trees shall be removed without prior approval of the Owner's representative.


D.
Provide protection for trees within 15 feet of utility excavation.


E.
When services are to be run side by side, a common trench may be used, providing the vertical and horizontal separation between the various services are maintained and providing the methods of bedding and backfill meet the approval of the Architect.  Contractors involved shall make their own agreement as to the sharing of the cost for the common trenching and backfill work.


F.
Unless otherwise noted on plans, all trenching and backfill for ducts shall be open cut



from the surface.

3.3
PROTECTION


A.
Each Contractor responsible for excavation work shall be required to provide all necessary barricades, fencing, bracing, shoring, sheet piling, warning signs, pumping, etc., for the protection of workers, general public and properties.  Excavation work shall comply with ASA Standard A10.2 "Safety Code for Building Construction" and AGC Standard, "Manual of Accident Prevention in Construction" and the Department of Labor Occupational Safety and Health Standards.

3.4
EXCESS EARTH FROM EXCAVATION


A.
Excess earth from excavation shall be removed from the site by the Contractor making the excavation, or may be piled on the site at a location designated by the Owner's Representative.


B.
Debris, trash or rock not usable for fill shall be removed from the site by each Contractor.


C.
Each Contractor shall be responsible for immediate "clean-up" of streets, roadways, and private property due to excavation.

3.5
RESTORATION OF EXISTING CONDITIONS


A.
When the excavation is within the area of finished site work by the Contractor, backfill to the height of rough grade.  Final surfacing by Contractor.

3.6
PROTECTION OF WORK


A.
Each Contractor shall be responsible for the protection of his work against being damaged by others during construction.

3.7
COMPACTION


A.
All backfill materials shall be clean and free from trash, debris, organic material and rocks (over4").  Compact in 6" lifts to 95% of compaction per the proctor test.  Hand compact all materials within 18" of conduits, pipes, etc.

END OF SECTION

La Quinta Inns, Inc.
16012-1
May 11, 2001

