
SECTION 16140PRIVATE 

WIRING DEVICES

PART 1
GENERAL

1.1
SUMMARY


A.
Section Includes:


1.
Switches.


2.
Dimmers.


3.
Convenience outlets.


4.
Coverplates.

1.2
GENERAL


A.
All wiring devices shall be of the type indicated below.  Color of devices and plates shall be white.

PART 2
PRODUCTS

2.1
MATERIALS


A.
Devices are based on Leviton, type Decora Plus.  Equivalent devices by General Electric, Pass and Seymour, and Hubbell are acceptable.

2.2
SWITCHES


A.
Rocker type switches shall be UL listed and conform to NEMA standards, as well as the latest Federal Specifications W-S896d.


1.
Rocker type , single pole
-
5621-W


2.
Rocker type , double pole
-
5622-W


3.
Rocker type , three-way
-
5623-W


4.
Rocker type , four-way

-
5624-W


5.
Key operated
-
1121-2L-W


6.
Pilot lighted (at guestroom entry and toilet room doors only)
-
5631-W


7.
Pilot lighted (at two room suite between living and 



bedroom, at toilet doors, and at entry door.

-
5631-2W &






5633-2W

2.3
DIMMERS


A.
Dimmers shall be UL listed, solid state, voltage compensated.

2.4
CONVENIENCE OUTLETS


A.
Conventional type, duplex, 3 wire, grounding type, side and backwired Leviton 16352-W.


B.
Exterior type, duplex, 3 wire, ground-fault interrupter (feed-through of end-of-line) with waterproof coverplate. Leviton 6899-W.  Device shall be mounted horizontally in a single gang, 3" deep outlet box.


C.
Isolated ground type, 3 wire, side and backwired, orange, Leviton 16362-IG duplex or Leviton 5361-IG simplex.

2.5
MANUAL MOTOR SWITCHES


A.
Single phase motors, smaller than 1/2 HP, shall be Allen Bradley Bullet No. 600 or Square D Class 2510, toggle switch type with neon pilot light.


B.
Provide appropriate NEMA enclosure depending on the application.  All units in finished areas shall be flush mounted and shall have special engraved coverplate matching coverplates specified for wiring devices. Coverplate shall specify unit controlled.


C.
Unless otherwise noted on the drawings as manual motor starters, controllers for a single phase loads 1000 watts and smaller shall be 20A AC switches with pilot light.

2.6
COVERPLATES


A.
General - Coverplates shall accommodate the devices installed in the outlet boxes.  When more than one device is indicated at the same location, a gang plate shall be used.  Plates covering boxes for future systems shall be blank.  All plates shall be compatible with the device configuration.  Install in each and every outlet box, as indicated on the drawings, a wiring device and coverplate.


1.
Interior or Exterior –Leviton 8000 Series.


2.
Block and Brick Walls - On block and brick walls (finished and unfinished areas) shall be of the materials specified above; however, all plates shall be "jumbo" size.


3.
Convenience outlets mounted in brick walls shall be mounted in horizontal position and located at nearest joint.


B.
Finish:


1.
In all finished areas, plate shall be 4 1/2" high, standard width white colored nylon with brass or stainless steel semi-flush head fastening screws and in exposed service conduit areas, cadmium plated pressed steel plates shall be used.  At locations where more than one switch and/or outlet occurs, coverplates shall be ganged or combination type as required.  Screws to be finished white to match cover plate.

2.7
GUESTROOM SMOKE DETECTORS


A.
Guestroom Smoke Detectors:  Wall mounted, 120VAC with batery backup, with intergal horn.  Integral horns - 85dB steady tone at 10 feet from the detector.  


1.
Manufacturer/model - Gentex 7100 or approved equal.

PART 3
EXECUTION

3.1
INSTALLATION


A.
Installation shall be in complete accordance with latest applicable edition of the NEC.  Mounting heights shall be in accordance with Section 16018 of the specification herein and as indicated on Drawings.

END OF SECTION

WIRING DEVICES
16140-3

