
SECTION 02820

FENCES AND GATES

PART 1 - GENERAL

1.01 SUMMARY

A. Provide aluminum fencing and gates for site perimeter and areas requiring separation.

1.02 SUBMITTALS

A. Product Data:  Submit manufacturer's product data and installation instructions for each material and product used. 

B. Samples:  Submit two representative samples of each material specified indicating visual characteristics and finish.  Include range samples if variation of finish is anticipated.

1.03 QUALITY ASSURANCE

A. Comply with governing codes and regulations.  Provide products of acceptable manufacturers which have been in satisfactory use in similar service for three years.  Use experienced installers.  Deliver, handle, and store materials in accordance with manufacturer's instructions.

PART 2 - PRODUCTS

2.01 MATERIALS

A. Framework:

1. Aluminum, ASTM B 429, Schedule 40, alloy 6063-T6, mill finish.

B. Gates:

1. Swinging type.

PART 3 - EXECUTION

3.01 INSTALLATION

A. Install materials in accordance with manufacturer's instructions and approved submittals.  Comply with ASTM F 567.  Install materials in proper relation with adjacent construction and with uniform appearance.  Coordinate with work of other sections.  Install posts to depth to avoid frost heave. 

B. Restore or replace damaged components.  Clean and protect work from damage.

END OF SECTION


FENCES AND GATES


02820-1

