
SECTION 06175

WOOD TRUSSES

PART 1 - GENERAL

1.01 SUMMARY

A. Provide prefabricated and pre-engineered wood trusses.

1.02 SUBMITTALS

A. Product Data:  Submit manufacturer's product data and installation instructions for each material and product used. 

B. Shop Drawings:  Submit shop drawings indicating material characteristics, details of construction, connections, and relationship with adjacent construction.

1. Shop drawings shall be prepared and stamped by a qualified engineer licensed in the jurisdiction of the project.

1.03 QUALITY ASSURANCE

A. Comply with governing codes and regulations.  Provide products of acceptable manufacturers which have been in satisfactory use in similar service for three years.  Use experienced installers.  Deliver, handle, and store materials in accordance with manufacturer's instructions.

B. Standards:  TPI, Design Specification for Metal Plate Connected Wood Trusses; TPI, Design Specification for Metal Plate Connected Parallel Chord Wood Trusses.

C. Design Engineering:  Registered engineer.

D. Fire-Retardant Treatment:  AWPA C20 for lumber; noncorrosive type.

PART 2 - PRODUCTS

2.01 MATERIALS

A. Wood Trusses:

1. Lumber Standard:  PS 20 American Softwood Lumber Standard.

2. Species:  Manufacturer's option.

3. Moisture Content:  Seasoned, 19 percent maximum.

B. Connectors, Fasteners, and Metal Framing Anchors:

1. Nails, Wire, Brads, and Staples:  FS FF-N-105.

2. Power Driven Fasteners:  National Evaluation Report NER-272.

3. Wood Screws:  ANSI B18.6.1.

4. Lag Bolts:  ANSI B18.2.1.

5. Bolts:  ASTM A 307, Grade A; ASTM A 563.

6. Metal Framing Anchors:  Hot-dip galvanized steel sheet, ASTM A 653, G60.

7. Connectors:  Hot-dip galvanized steel sheet, ASTM A 653, G60.

PART 3 - EXECUTION

3.01 INSTALLATION

A. Comply with recommendations of TPI Design Specifications for Metal Plate Connected Wood Trusses.

B. Install materials and systems in accordance with manufacturer's instructions and approved submittals.  Install materials and systems in proper relation with adjacent construction.  Coordinate with work of other sections.

C. Restore damaged components.  Clean and protect work from damage.

END OF SECTION


WOOD TRUSSES


06175-1

