
SECTION 06400

ARCHITECTURAL WOODWORK

PART 1 GENERAL

1.1 SUMMARY

A. Provide interior architectural woodwork:

Standing and running trim and rails.

1. Casework and countertops.

PART 2 PRODUCTS

2.1 MATERIALS

A. Interior Standing and Running Trim and Rails:

Species for Opaque Finish:  Any closed-grain hardwood or medium density fiberboard.

1. Grade:  Paint.

B. Interior Plastic Laminate Clad Casework:

Laminate:  High pressure decorative laminate, NEMA LD-3.

1. Grade:  Custom.

2. Face Style:  Reveal overlay.

3. Frame Fabrication:  Face frame.

C. Casework Hardware and Auxiliary Materials:

Hardware Standard:  ANSI/BHMA A156.9.

1. Hardware Finish and Base Metal:  Satin Brass.

D. Interior Plastic Laminate Clad Countertops:

Laminate:  High pressure decorative laminate, NEMA LD-3.

1. Grade:  Custom.

2. Core:  Plywood.

3. Edge:  Laminate.

E. Solid Surfacing Material Countertops:

Type:  Synthetic countertops.

1. Grade:  Custom.

2. Edge: Decorative.

F. Shelving Specialties:

Shelving: Plywood with hardwood edgeboard.

1. Closet Rods: Chrome plated steel.

G. Auxiliary Materials:

Screws:  FS FF-S-111.

1. Nails:  FS FF-N-105.

2. Anchors:  Type required for secure anchorage.

END OF SECTION


ARCHITECTURAL WOODWORK


06400-1

