
SECTION 07210

BUILDING INSULATION

PART 1 GENERAL

1.1 SUMMARY

A. Provide building insulation and vapor retarders as required.

PART 2 PRODUCTS

2.1 MATERIALS

A. Thermal Board Insulation:

1). Application:  Foundation walls.

2). Type:  Polyisocyanurate board, rigid, FS HH-I-1972/1, Class 2.

B. Thermal Blanket/Batt Insulation:

1). Application:  Thermal insulation in studs in exterior walls.

2). Application:  Thermal insulation at underside of roofs, over heated spaces and over soffits.

3). Type:  Glass fiber or mineral slag fiber, ASTM C 665, Type I (unfaced).

4). Type:  Glass fiber or mineral slag fiber, ASTM C 665, Type III (foil-scrim-kraft vapor-retarder membrane).

C. Thermal Loose Insulation (blown-in)

1). Application:  Thermal insulation above top floor ceilings in structural space.

2). Type:  Glass fiber loose insulation FS HH-1-1030B Type I, UL Listed.

D. Accessories:

1). Adhesives and mechanical anchors and clips.

2). Crack sealers and tapes.

PART 3 PART 3 – EXECUTION

3.1 INSTALLATION

A. Install materials and systems in accordance with manufacturer's instructions and approved submittals.  Install materials and systems in proper relation with adjacent construction.  Coordinate with work of other sections.  Provide full thickness in one layer over entire area, tightly fitting around penetrations.

B. Protect installed insulation.

END OF SECTION


BUILDING INSULATION


07210-1

