
SECTION 07550

MODIFIED BITUMINOUS MEMBRANE ROOFING

PART 1 - GENERAL

1.01 SUMMARY

A. Provide modified bituminous membrane roofing.

1.02 SUBMITTALS

A. Product Data:  Submit manufacturer's product data and installation instructions for each material and product used. 

B. Shop Drawings:  Submit shop drawings indicating material characteristics, details of construction, connections, and relationship with adjacent construction.

C. Warranty:  Submit manufacturer’s standard warranty.  Include labor and materials to repair or replace defective materials.

1. Warranty Period:  10 years.

1.03 QUALITY ASSURANCE

A. Comply with governing codes and regulations.  Provide products of acceptable manufacturers which have been in satisfactory use in similar service for three years.  Use experienced installers.  Deliver, handle, and store materials in accordance with manufacturer's instructions.

B. Listing:  UL Class A external fire exposure:

C. Listing:  FM Class I construction.

PART 2 - PRODUCTS

2.01 MATERIALS

A. Manufacturers: Firestone Building Products Co., Celotex Corp., GAF Building Materials, Supreme Roofing and Waterproofing or approved equal.

B. Modified Bituminous Sheet Roofing:

1. Type:  Fully adhered.

2. Modifier:  Styrene-butadiene-styrene (SBS).

C. Auxiliary Materials:

1. Vapor Retarder: Bituminous vapor retarder.

2. Insulation:  Perlite board.

3. Insulation:  Polyisocyanurate-foam board.

4. Insulation:  Composite insulation board.

5. Insulation:  Cellular-glass board.

6. Surfacing Aggregate:  Crushed stone, free of sharp edges, ASTM D 1863.

7. Roof Coating:  Asphalt emulsion, ASTM D 1227.

8. Walkway Protection Boards:  Compatible with system.

9. Sheet Metal Accessories:  SMACNA and NRCA recommendations.

PART 3 - EXECUTION

3.01 INSTALLATION

A. Inspect substrate and report unsatisfactory conditions in writing.  Beginning work means acceptance of substrate.  Coordinate installation with other trades, including carpentry, flashing and penetrating work.

B. Comply with NRCA Roofing and Waterproofing Manual and manufacturer's installation instructions.

C. Clean, prime and prepare substrate.

D. Install insulation with tightly butted joints and neatly fitted around penetrations.

E. Begin roof installation only in presence of manufacturer's representative.

F. Install walkway protection membrane at locations indicated and where required to provide access to roof mounted equipment.

G. Restore or replace damaged components.  Protect work from damage.

END OF SECTION

MODIFIED BITUMINOUS MEMBRANE ROOFING


07550-1

