
SECTION 07900

JOINT SEALERS

PART 1 GENERAL

1.1 SUMMARY

A. Provide joint sealers at interior and exterior vertical and horizontal joints.

B. Field-Constructed Mock-Ups:  Each joint type.

PART 2 PRODUCTS

2.1 MATERIALS

A. Urethane Elastomeric Joint Sealants:

Manufacturers: Pecora Corp., Sika Corp., Tremco or approved equal.

1. Type and Application:  Multi-part nonsag urethane sealant, ASTM C 920:

a. Application:  For joints in horizontal. surfaces.

b. Exterior use.

B. Silicone Elastomeric Joint Sealants:

Manufacturers: Dow Corning, GE Silicones, Tremco, or approved equal.

1. Type and Application:  One-part nonacid-curing silicone sealant, ASTM C 920, modulus as required for application:

a. Application:  For joints in vertical surfaces.

b. Exterior use.

C. Latex Joint Sealants:

Manufacturers:  Pecora Corporation, Polymeric Systems, Inc., Sonneborn Building Products, Tremco, or approved equal.

1. Type:  Acrylic-emulsion, ASTM C 834.

2. Application:  Interior joints in vertical and overhead surfaces with limited movement.

D. Compression Seals:

1. Type:  Preformed hollow neoprene gasket, ASTM D 2628.

2. Application:  Wide exterior joints in vertical surfaces.

E. Fire-Resistive Joint Sealers:

Type:  One part fire-stopping sealant.

1. Application:  Penetrations in fire-rated floor and wall assemblies.

F. Paving Joint Fillers:

Type:  Bituminous fiber.

1. Application:  Filler for exterior paving joints.

G. Auxiliary Materials:

Plastic foam joint fillers.

1. Elastomeric tubing backer rods.

2. Bond breaker tape.

END OF SECTION


JOINT SEALERS


07900-1

