
SET 1

Door #
EXTERIOR FROM LOBBY

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"
NRP
US4
633
McKinney

1 ea.
Reinforcing Pivot

B1923




US2C
663
Markar

1 ea.
Exit Device


21-8888 F x 713-8 ETL


US4
606
Sargent

1 ea.
Closer



351-CPS



EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Threshold


TH015A
36"


US27
627
Ultra

1 set
Weatherstrip


WS165AV
1-36" x 2-84"

US27
627
Ultra

1 ea.
Sweep



DB006CN
36"


US28
628
Ultra

1 ea.
Drip Cap


WS025C
40"


US28
628
Ultra

SET 2

Door #
EXTERIOR FROM STAIR


EXTERIOR FROM LOBBY


EXTERIOR FROM BREAKFAST AREA

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"
NRP
US4
633
McKinney

1 ea.
Reinforcing Pivot

B1923




US2C
663
Markar

1 ea.
Exit Device


8810 F ETL



US4
606
Sargent

1 ea.
Electric Strike


310-4




US4
606
Folger Adam

1 ea.
Closer



351-CPS



EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Threshold


TH015A
36"


US27
627
Ultra

1 set
Weatherstrip


WS165AV
1-36" x 2-84"

US27
627
Ultra

1 ea.
Sweep



DB006CN
36"


US28
628
Ultra

1 ea.
Drip Cap


WS025C
40"


US28
628
Ultra

NOTE:  Wall mount card reader by owner.

SET 3
Door #
EXTERIOR FROM PANTRY


EXTERIOR FROM ELECTRICAL

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"
NRP
US4
633
McKinney

1 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

1 ea.
Closer



351-CPS



EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Threshold


TH015A
36"


US27
627
Ultra

1 set
Weatherstrip


WS165AV
1-36" x 2-84"

US27
627
Ultra

1 ea.
Sweep



DB006CN
36"


US28
628
Ultra

1 ea.
Drip Cap


WS025C
40"


US28
628
Ultra

SET 4

Door #
CORRIDOR FROM MEETING ROOM


CORRIDOR FROM MEETING/BREAKFAST (OVERFLOW)


CORRIDOR FROM MEETING
Each pair to have:
6 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Exit Device


12-21-PP8613 ETL
36" x 84"
US4
606
Sargent

1 ea.
Exit Device


12-21-PR8613 ETL
36" x 84"
US4
606
Sargent

2 ea.
Closer



1430-P9



EAB
696
Sargent

2 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

2 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

2 ea.
Electro-Mechanical Hold Open
FM998




SG
696
Rixson

20 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

NOTE:
No astragal, no coordinator, for fire doors listed by UL for this use.

SET 5

Door #
CORRIDOR TO STAIR

Each door to have:
3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Exit Device


12-8888 F x 715-8 ETL


US4
606
Sargent

1 ea.
Closer



1430-O




EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Electro-Mechanical Hold Open
FM998
First floor only


SG
696
Rixson

17 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

SET 6

Door #
CORRIDOR TO MAIN LAUNDRY


FRONT OFFICE TO VOD


CORRIDOR TO MAINTENANCE


CORRIDOR TO STORAGE


CORRIDOR TO JANITOR


CORRIDOR TO PANTRY


ELEVATOR LOBBY TO LINEN


CORRIDOR TO STORAGE (8TH FLOOR)


CORRIDOR TO MECHANICAL
Each door to have:
3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

1 ea.
Closer



1430-O




EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

17 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

SET 7

Door #
CORRIDOR TO BOARD ROOM

Each door to have:
3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Entrance Lockset

21-28-10G05 LL


US4
606
Sargent

1 ea.
Closer



1430-O




EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Wall Stop


409




US4
606
Rockwood

17 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

SET 8

Door #
CORRIDOR TO BUSINESS CENTER

Each door to have:
3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Entrance Lockset

21-28-10G05 LL


US4
606
Sargent

1 ea.
Closer



1430-O




EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Floor Stop


441




US4
606
Rockwood

17 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

SET 9

Door #
CORRIDOR TO EXERCISE ROOM


CORRIDOR TO GUEST LAUNDRY

Each door to have:
3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Lockset supplied by owner.

1 ea.
Closer



1430-O




EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

17 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

SET 10

Door #
LAUNDRY FROM LAUNDRY CHUTE

Each door to have:
3 ea.
Hinges



T4A3786
5" x 4.5"

US4
633
McKinney

1 ea.
Passage Latchset

28-10U15 LL



US4
606
Sargent

1 ea.
Closer



1430-O




EAB
696
Sargent

1 ea.
Wall Stop


406




US4
606
Rockwood

18 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

SET 11

Door #
LOBBY TO FRONT OFFICE

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

1 ea.
Closer



1431-O




EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 12

Door #
LOBBY TO LUGGAGE

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 13

Door #
FRONT OFFICE TO OFFICE, LAUNDRY TO HOUSEKEEPING

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Entrance Lockset

21-28-10G05 LL


US4
606
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


409




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 14

Door #
LAUNDRY TO TOILET

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Privacy Latchset

28-10U65 LL



US4
606
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


409




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 15

Door #
CORRIDOR TO MEN


CORRIDOR TO WOMEN

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Passage Latchset

28-10U15 LL



US4
606
Sargent

1 ea.
Closer



1431-O x DA



EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 16

Door #
FRONT OFFICE TO/FROM FRONT DESK


CORRIDOR TO VENDING
Each door to have:

NOTE:
Cased opening - No hardware required.

SET 17

Door #
CORRIDOR TO GUEST ROOM

Each door to have:
3 ea.
Spring Hinges


1502

4.5" x 4.5"

US4
633
McKinney

1 ea.
Lockset residential, by approved card key access manufacturer.

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

17 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

1 ea.
Door Viewer 190o

622




US4
606
Rockwood

1 ea.
Security Door Guard

603




US3
605
Rockwood

SET 18

Door #
GUESTROOM TO TOILET

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Privacy Latchset

28-10U65 LL



US4
606
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood



mounted on door @ tub level

3 ea.
Silencers


608




GREY

Rockwood

SET 19

Door #
GUEST ROOM FROM CLOSET

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Roller Latch


592




US4
606
Rockwood

1 ea.
Single Dummy Lever

7U93 LL



US4
606
Sargent

1 ea.
Mop Plate


6" x 23" x .050"



US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 20

Door #
GUEST ROOM CONNECTING DOORS

Each set of doors to have:

6 ea.
Spring Hinges


1502

4.5" x 4.5"

US4
633
McKinney

2 ea.
Exit Latchset


7G15-3 LL



US4
606
Sargent

2 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

2 ea.
Floor Stop


406




US4
606
Rockwood

34 ft.
Smoke Seal


WS280D



DK BNZ
Ultra

2 ea.
Door Viewer 190o

622




US4
606
Rockwood

SET 21

Door #
LIVING TO GUESTROOM

Each pair of doors to have:

6 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

2 ea.
Privacy Latchset

28-10U65 LL



US4
606
Sargent

2 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

2 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

2 ea.
Wall Stop


406




US4
606
Rockwood

6 ea.
Silencers


608




GREY

Rockwood

SET 22

Door #
STORAGE BUILDING / EXTERIOR

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"
NRP
US4
633
McKinney

1 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

1 ea.
Closer



351-CPS



EAB
696
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Threshold


TH015A
36"


US27
627
Ultra

1 set
Weatherstrip


WS165AV
1-36" x 2-84"

US27
627
Ultra

1 ea.
Sweep



DB006CN
36"


US28
628
Ultra

1 ea.
Drip Cap


WS025C
40"


US28
628
Ultra

SET 23

Door #
MEETING ROOM TO STORAGE

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

1 ea.
Wall Stop


406




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 24
Door #
FRONT OFFICE TO EMP. BREAKROOM

Each door to have:

3 ea.
Hinges



TA2714

4.5" x 4.5"

US4
633
McKinney

1 ea.
Passage Latchset

28-10U15 LL



US4
606
Sargent

1 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

1 ea.
Mop Plate


6" x 35" x .050"



US4
606
Rockwood

1 ea.
Wall Stop


406




US4
606
Rockwood

3 ea.
Silencers


608




GREY

Rockwood

SET 25

Door #



EXTERIOR FROM MECHANICAL

Each pair of doors to have:

6 ea.
Hinges



TA2714

4.5" x 4.5"
NRP
US4
633
McKinney

2 ea.
Storeroom Lockset

21-28-10G04 LL


US4
606
Sargent

2 ea.
Closer



351-CPS



EAB
696
Sargent

2 ea.
Kick Plate


10" x 34" x .050"


US4
606
Rockwood

2 ea.
Threshold


TH015A
36"


US27
627
Ultra

2 set
Weatherstrip


WS165AV
1-36" x 2-84"

US27
627
Ultra

2 ea.
Sweep



DB006CN
36"


US28
628
Ultra

2 ea.
Drip Cap


WS025C
40"


US28
628
Ultra

END OF HARDWARE SETS

1
1
PAGE


