STRUCTURAL INSPECTION

Date:
April 28, 2006
Type:
Residential property
RE:
Post Hurricane Katrina Structural Inspection

Structure:
Single story wood frame/concrete foundation/asphalt shingle roof/brick and vinyl siding 
Owner(s): 
Stephen Ladner




       
176 Sun Valley








Slidell, LA







985-788-2696
History:

Mr. Ladner contacted Dammon Engineering to perform a structural inspection on the referenced residence.  He was concerned about the structural integrity of the home after Hurricane Katrina.
Findings:

During a visual inspection of the home, the following was noted:

1. The roof has recently been replaced.
2. The southwest corner of the brick ledge is fractured.
3. The soffit on the west side is pulled down.
4. The siding on the rear of the home is separated on the northwest corner.
5. The siding on the rear of the home is torn near the center.

6. The soffit on the northeast corner is pulled down.

7. There is a broken window on the east side of the home.

8. The vinyl siding is pulled away from the southeast corner of the home.

9. The water rose above the power distribution panel 

10. Wire, receptacles and switches were below the water line.

11. A new electric water heater has been installed.

12. The interior wall and ceiling sheetrock and insulation have been removed.
13. The front and back doors need to be replaced.
Analysis

Minor repairs to the exterior of the home are required.  Replacement of the exterior doors, interior doors, insulation, wall board, electrical outlets, switches, electrical panel and wiring below the water line, bath room and kitchen fixtures, and floor coverings are required to finish the home.
Conclusion:

It is my opinion that this home can be repaired and will be structurally sound and in good condition once all of the repairs have been completed.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796
	
	 


[image: image1.png]C
— Gy | f30008Z

30 291157
291092
28212 25/15Z

261152 ‘241152
271212 23212
20 80 70

e

Track of Hurricane Katrina
@ o 2330 August 2005




Tracking Chart for Hurricane Katrina
Data courtesy the US National Hurricane Center

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


