STRUCTURAL INSPECTION

Date:
February 16, 2006
Type:
Residential
RE:
Post-Hurricane Katrina Structural Inspection

Structure:
Wood frame on concrete foundation/Vinyl exterior/Brown metal roof 
Owner(s): 
Miriam Pate
Property: 
2303 Pelican


Slidell, La 



985-768-8803/985-892-9122

History:

Mrs. Pate contacted Dammon Engineering to perform a structural inspection on the above residence.  She was concerned about damage to the structure of the home after hurricane Katrina’s high winds.  She stated as many as three trees hit her home.  
Findings:

After a visual inspection of the home, the following was noted:

1. There is water damage in the kitchen and laundry room.
2. The seams in the living room vaulted ceilings are separating
3. The archway corner seams between the living room and kitchen are separated.

4. The fireplace cap is leaking and the owner states she can hear water dripping inside the chimney chase after a rainstorm.

5. Water damage to the master bedroom walls and ceilings.

6. Water damage to the ceilings in the master bathroom and the second bedroom up stairs.

7. An attic inspection shows that roof rafters are pulled away on the south side of the roof beam. 
8. The metal roof screws are no longer set at the required installation specification and allow water to continue to enter the home.

Analysis:

1. Repairs to the roof and supporting structure should be accomplished as soon as possible to prevent further damage. (see additional notes below).  The roof metal should be removed and the wood framing reset and the metal roof reinstalled with new fasteners, following the manufacturer’s specifications.
2. The damaged sheetrock should be removed, including the insulation surrounding the area, (see additional notes below) then replaced.
Conclusion:

The damage to the structure of the home is a direct result of the high winds and trees hitting the home.
There may be additional damage not seen until the coverings are removed.
It is my professional opinion that once the above repairs are accomplished, this dwelling will be structurally sound.

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Code requirements for that work.

Information homeowners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


