DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS					ENGINEERS

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail				   					     985-649-5832

Slidell, LA 70458								            FAX 985-641-5950

# Structural Inspection

February 17, 2012

For: 	Hilman & Linda Bertrand

1337 Westlawn Dr.

Slidell, Louisiana 70460

Construction:

Two-story, wood frame with brick/vinyl veneer and composition shingle roof on a conventional foundation, the home is approximately 30 years.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence.  No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. & Mrs. Bertrand contacted Dammon Engineering to request a structural inspection of their home. They are concerned about possible settlement of the addition in the rear of the home after a home leveling company leveled it.

Findings:

The addition of the home has clearly settled.

The very rear of the addition has settled causing the foundation of the addition, near the existing foundation of the home, to rise up, consequently cracking the ceramic tile and sheetrock at the location where the addition was added onto the existing residence.  This settlement has pulled the crown molding away from the sheetrock ceiling and the base board molding away from the sheetrock walls.

No corresponding crack(s) or other visible separation was noted in the brick veneer, except where the addition was attached to the existing residence.  At this location, there is visible separation between the addition and the existing residence.

The flooring on the interior of the residence, both in the addition and in the existing residence, appears to be in good condition in all locations, except where the addition joins the existing residence.

The sheetrock walls and ceilings appear to be square and no signs of stress or cracks were noted to show any other stress, except where the addition joins the existing residence.

The interior and exterior of the existing home was inspected and no cracking of the foundation, brick veneer, flooring or sheet rock was noted in the existing residence.

Conclusion:

Once the foundation of the addition is properly re-leveled to the appropriate height and the crack closes in the foundation, any remaining visible foundation cracks should then be sealed with one of the commercial products designed for that purpose.  A non-shrinking mortar, or caulk, should be used to fill any openings in the exterior brick to prevent insects from entering the residence.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Attachment(s):		Interior and Exterior Photographs