DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

January 10, 2007
STRUCTURAL CERTIFICATION

For:
Woody and Lori Dahl
Location:
66 Talbot Ln


Mandeville, La 73396
To Whom It May Concern:

Building Description: Wood frame, post tension concrete foundation, composition shingle roof, and brick veneer on four sides. The residence is approximately 3,396 sq. ft. and fifteen years old.
Scope: 

This inspection is limited to a visual inspection of the interior and exterior walls, ceilings, floors, roof and foundation.  No inspection of the mechanical or electrical systems was performed.

History:

Mr. Dahl contacted Dammon Engineering to perform a structural inspection on the referenced residential property.  Mr. Dahl is considering purchasing the home and his home inspection noted a crack in the foundation, which he wished to have inspected by a professional engineer.
Findings:

This inspection covered visible components (foundation, and wall) and their ability to support the home.

A visible crack is seen in the foundation on the right side of the home near the bathroom window.  The crack appears to travel through the foundation and across to the A/C return air filter grill inside the hallway of the home and past that point.  Wood and carpeted floors prevent further inspection.  No cracks in the walls, ceilings or brick façade are visible.  The doors and windows open and close normally.
There is a visible crack in the brick ledge at the front right corner of the house.

Analysis:

The post tension foundation is designed to hold the foundation together as the foundation settles.  The crack is due to settlement and likely happened during the first five years of its life.  Further settlement is unlikely because of the age of the home.

The crack in the brick ledge is common for concrete foundations and is of no structural concern.
Conclusion:

I certify the above residence is structurally sound.
Sincerely,

Emmett Dammon

La. License 8796
