554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

December 10, 2018

For: Karl Holzenthal

616 Jefferson Ave. Apt 3

Brooklyn, NY. 11221

Ref: 129 Nottingham Drive

Slidell, La.

Construction:

One-story, wood frame, brick veneer, ceramic, wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to conduct a structural inspection of the referenced home due a previous inspection that noted elevation differences in the foundation.

Findings:

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up at the front of the living room at zero inches and measured throughout the home. It is important to note that all floor height conditions vary due to ceramic height and wood flooring height.

An overall visual inspection of the exterior of the home was conducted and several items were noted. There is a hairline crack in the brick veneer on the left side of the home that runs from the middle of the wall down to the foundation along with other hairline cracks but these cracks do not go into the foundation. Hairline cracks in the brick veneer on the front of the home were seen as well but do not go into the foundation. There are hairline cracks in the brick veneer on the right side of the home but do not go into the foundation.  Some of these cracks have been repaired in the past.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc. The brick wall is a veneer only and does not provide any structural support.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water around the foundation.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Seal the cracks in the brick veneer with a non-shrinking grout, to prevent insects from entering the home.

- Gutters and down spouts are needed to direct the rainwater away from the foundation and out to the yard. Existing down spouts need to be directed away from the foundation/patio and into the yard.

Sincerely,

Brian Mistich, P.E.