554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

October 24, 2017

For: 	Rachel Jacobs

212 Melody Lane

Slidell, La. 70458

Ref:     212 Melody Lane

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a conventional foundation.  Home is approximately 35 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection that noted some cracks in the brick veneer and stated a slope in the foundation.

Findings:

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up at the front of the home near the dining room. The slopes range in the house from 0.0” to -2.3” in the main areas. The area in question (bedrooms), from the back of the master bedroom to the front bedroom has a combined slope of -4”.

An overall visual inspection of the exterior of the home was conducted and couple items were noted. There is a crack in the brick veneer and mortar on the left side of the home near the front bedroom above the window but does not show up in the foundation. The garage wall near the entrance of the house, there is a crack in the brick veneer and mortar approximately 4’ long that does not show up in the foundation.

In the rear of the home near the breakfast room, the bricks appear to be replaced as it is understood a sliding door was replaced with a solid wall.

It was also noted that there are no gutters or down spouts on the home. The front corner of the home near the entrance and garage, the earth is eroding around the foundation due to the water off runoff of the roof.  In the rear of the home, the corner of the family room and master bedroom the earth is eroding around the foundation due to water runoff of the roof.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

Due to the fact that there are no visible signs of distress in the walls, ceilings or wood/ceramic flooring in the house at the time of this inspection, it is likely that settlement has subsided. Cracks may propagate in the future; however, at this time it appears stable and does not appear to compromise the integrity of the house.

It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

1)  Seal the cracks in the brick veneer with a non-shrinking grout, to prevent insects from entering the home.

2) The yard should also be re-graded to allow for proper drainage of rainwater away from the foundation and add gutters and down spouts to carry the rain water to the street.

Sincerely,

Brian Mistich, P.E.