554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

January 11, 2023

John & Stefanie Ascherl

226 Hoffman Lane

Waveland MS. 39576

Construction:

Two-story, wood frame, board and batten veneer with a composition shingle roof on a pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due some noted cracking in the pier and beam foundation.

Findings:

An overall visual inspection of the exterior of the foundation was conducted and a few items were noted.  There are a few hairline cracks showing up along with some of the wire mesh protruding and showing up as rust stains.

Outside of the foundation, the natural grade is below the foundation and in some areas slope back to the foundation.

Conclusion:

It is important to note that all foundations settle.  There are no gutters on this home to properly divert the rain water away from the foundation. Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, substandard design, tree roots, improper drainage, etc.

Recommendation:

The cracks in the surface finish do not appear to have compromised the structural integrity of the home.  It appears that the structural integrity of this home is currently sound. The foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

- Seal the cracks in the concrete finish with a non-shrinking grout.

- Bring in fill and slope away from the house and install sod.

2) Gutters and down spouts should be added to allow proper drainage of rainwater away from the foundation.

Sincerely,

Brian Mistich, P.E.