554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

December 10, 2019

Jimmie Scott

North Shore Auctions LLC.

37505 Brownsvillage Rd

Slidell, LA 70460

It was requested to inspect a newly purchased building for removal of four walls in the front office area. This will allow more occupants during auction time.

The area in question is located in front of the bldg with offices and storage above it. The walls are 2x6 wood studs with 2x6 top plates and 2x12 ceiling joist wrapped in sheetrock for a rated ceiling. These are load bearing walls.

The 2nd floor flooring (3/4” plywood with 5/8” sheetrock) was removed to verify which way the ceiling joist are installed. The ceiling joist run North to South in the offices and East and West in the foyer.

The contractor shall install two 2x12’s under each load bearing wall, glued and thru bolted to support each ceiling joist in both directions with a 6x6 post in the middle and 4x4 posts at the ends.

Sincerely,

Brian Mistich, P.E.