554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

October 2, 2017

For: 	Donna Schmalz

58233 Loblolly Lane

Slidell La. 70460

Ref:     443 Pine Cone Lane

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a conventional foundation.  Home is approximately 35 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection that noted some cracks in the brick veneer.

Findings:

The interior of the home (master bedroom) was inspected where the bricks have cracked on the exterior wall and no signs of repairs were noted in the walls or ceiling.

An overall visual inspection of the exterior of the home was conducted and couple items were noted. There is a crack in the brick veneer and mortar on the left side of the home near the hose bib that runs about five feet above it and down to the foundation but does not show up in the foundation. Between the hose bib and the rear wall there is a crack in the brick veneer and mortar that does not show up in the foundation.

It was also noted that the earth is eroding around the foundation due to the down spouts hitting directly on the ground and flowing to the front of the home.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

1)  Seal the cracks in the brick veneer with a non-shrinking grout, to prevent insects from entering the home.

2) The yard should also be re-graded to allow for proper drainage of rainwater away from the foundation or add extensions onto the existing down spouts.

Sincerely,

Brian Mistich, P.E.