554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Re-Structural Inspection

November 03, 2022

For: 	Troy & Jeanell Bordelon

445 Evergreen Ct.

Slidell La. 70458

Construction:

One-story, wood frame, brick and vinyl veneer, ceramic and wood flooring with a composition shingle roof on a post tension foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request another structural inspection of the referenced home due to a home inspection noting the cracks in the brick veneer on the right side of the home.

Findings:

A re-inspection of the right side of the home was conducted and the cracks noted in the brick veneer still remain.  These cracks are mostly concentrated around the electrical service and kitchen window area down to the waterline coming into the home and do not penetrate the foundation.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will continue to expand and contract with the ground movement and with any standing water.

Items to be addressed are as follows:

- Seal the cracks in the foundation with a non-shrinking grout, to prevent insects from entering.

- Remove any tress and roots within ten feet of the home.

- Provide proper drainage away from the home.

Sincerely,

Brian Mistich, P.E.