554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

September 27, 2022

For: 	Distinctive Real Estate, Inc.

1501 Gause Blvd. Ste. 1

Slidell, La. 70458

Ref:     Bayou Mason

4800 Pontchartrain Drive

Slidell. La. 70458

Construction:

Two-story, wood frame, brick/vinyl veneer with a metal roof on a conventional piling foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the building, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a re-inspection of the referenced apartments. The contractor has removed some of the vinyl siding in the damaged areas that were noted prior.

Findings:

Upon inspection of the right staircase, some of the vinyl has been removed to expose the horizontal wood support that appears not be rotten. The wood columns in that area have rusted/deteriorated supports that have failed and allowed the collapse of the concrete walkways.

The vinyl soffit has been removed in the right hallway leading to the rear of the bldg. It has exposed the rotten plywood supporting the concrete walkway, rotten horizontal supports and rotten floor joist that need to be replaced.

Recommendation:

Items that need to be addressed are as follows:

- Remove the entire vinyl fascia covering the horizontal wood supports on all the upper walkways to detect rotten wood.

- Remove the entire vinyl and wood soffit from under the walkways to detect rotten wood.

- Remove and replace all connectors for the wood columns (ground floor and second floor).

- Remove and replace all horizontal walkway supports that are rotten.

- Remove and replace all floor joists that are rotten.

- Remove and replace all rotten plywood that is supporting the concrete walkways. Install new plywood with lightweight concrete.

- Remove and replace all cracked concrete walkways that have more than 1”separation or jack up back in place.

- Seal all concrete walkways.

- Reinstall all vinyl.

Sincerely,

Brian Mistich, P.E.

Pictures attached