554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

August 8, 2019

For: Luke Calamari

Gardner Realtors

1300 Gause Blvd Suite c-2

Slidell, La. 70458

Ref: 513 Cross Gates Blvd.

Slidell, La. 70461

Construction:

One-story, wood frame, brick veneer, ceramic, wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to conduct a structural inspection of the referenced home due a previous inspection that noted a separation in the fascia against the brick fireplace.

Findings:

A visual inspection of the exterior brick fireplace was conducted and you can see where the fascia and soffit doses not touch the brick fireplace at the top by at least a half inch. This separation appears to be where there was wood trim was installed and at some point and time removed. You can clearly see where the trim was located due to the excess paint on the brick fireplace.

Recommendation:

The separation in the fascia and soffit against the brick fireplace does not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound.

Items to be addressed are as follows:

- Install wood trim to cover the separation between the brick fireplace and fascia/soffit, caulk and paint to match.

Sincerely,

Brian Mistich, P.E.