554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

June 12, 2018

For: 	Donald Rouleau

543 Tanglewood Dr.

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer with stucco, with a composition shingle roof on a conventional foundation.  Home is approximately 4 1/2 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to perform a structural inspection of the referenced home due to some cracks in the brick veneer, doors not shutting properly and previous ceramic tiles that were cracked in the kitchen but have been since repaired.

Findings:

An overall visual inspection of the exterior of the home was conducted and a few items are noted.

There is a 12” hair line crack in the mortar and brick that runs vertically and stops before the grade beam on the right rear of the home. Some areas on this wall, mortar was noted to have been replaced as well as on the front of the home. No cracks in the foundation were noted.

Gutters and down spouts have been added but are not properly tied in to the subsurface drainage.

A visual inspection of the interior sheet rock ceilings and walls of the home where not noted to have any cracks in them. All interior floors have a covering on them and it was not inspected under them.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

1) Verify that the gutters and down spouts are connected properly and not clogged and will carry the rain water away from the foundation and out to the street.

2)  Seal cracks in the brick veneer and mortar with a non-shrinking grout, to prevent insects from entering the home.

Sincerely,

Brian Mistich, P.E.