554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

December 6, 2021

Lisa Collins

110 Bayou Paquet Street

Slidell, La. 70460

Construction:

One-story workshop, wood frame, vinyl veneer with a metal roof on a flat foundation.

Scope:

This inspection is limited to a visual inspection of the foundation of the workshop and screened in porch. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced one story workshop and screened in porch foundation to verify if it will support a new wood frame addition connecting to the house.

Findings:

Upon inspection, an overall visual inspection of the workshop foundation was completed and noted that three add-ons of the foundation were installed for this workshop. The two foundations in question do not have a concrete grade beam under them. One foundation is 6” thick and the other is 4” thick with no exterior grade beams.

Also, the screened in porch foundation was noted to have three big cracks in the foundation and was noted to only be 4” thick with no exterior grade beams.

Recommendation:

Items to be addressed are as follows:

- Remove the existing concrete foundations where needed and install a new concrete foundation with grade beams to match the new structure being built.

Sincerely,

Brian Mistich, P.E.