554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

July 15, 2020

For:	April Brown

RCG Ventures, LLC

3060 Peachtree Rd NW, Suite 400

Atlanta, Ga. 30305

Ref: 105 Northshore Blvd. Suite, 135. Village at Northshore – JoAnn – Building Damage

Construction:

One-story, prefab metal structure, brick exterior and cinder block interior, VCT flooring on a post tension foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the building, including the interior and exterior foundation of the building. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced building due to a vehicle accident were the vehicle hit the corner of the southeast side of the building.

Findings:

An exterior inspection of the southeast corner of the building noted fractured and 	cracked bricks 	extending approximately 36” high. The eastern corner side of the building was noted to have a hairline 	crack in three 	bricks.  The crack does not appear to propagate the fracture into foundation below.

Conclusion:

While the exterior of the building was damaged in that area, the interior of that area does not appears to suffer from any structural impairment

Recommendation:

Items to be addressed are as follows:

- Remove all damaged bricks in that area on both the southeast and eastern side of the building and replace with new to match existing.

Sincerely,

Brian Mistich, P.E.