DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

November 24, 2006

FRAMING AND ROOF INSPECTION

For:
Mrs. Zarinski

Location:
316 Margon Ct.



Slidell, La. 70458

Residence Description:
Wood frame, conventional concrete foundation, composite shingle roof, brick veneer

Scope:

This inspection is limited to a visual inspection of the framing and roof.  No inspection of the interior and exterior walls, ceilings, floors, foundation or mechanical or electrical systems was performed.

History:

Mrs. Zarinski contacted Dammon Engineering to perform an inspection on the referenced residential property.  Mrs. Zarinski stated that a contractor was hired to repair the roof, broken roof rafters and decking.  The residence is approximately thirty years old.

Findings:

This inspection covered visible components (roof framing and roof shingles) and their ability to support and protect the residence.

1. Roof drip edge and felt paper improperly installed.

2. Roof starter course improperly installed.

3. Heater roof jack damaged.

4. H clips not installed on plywood decking.

5. Damaged plywood decking.

6. Heater vent stack improperly installed.

7. Hip rafter was repaired improperly.

Analysis:

The roof drip edge is installed over the top of the felt paper and nailed to the roof decking. Removal of the drip edge and proper reinstallation is required.

The starter coarse shingle is nailed with an insufficient number of nails to meet the requirements of the IBC.

The roof jack for the heater was installed with the incorrect angle jack.  Replacement of the roof jack with one of the proper angle is required.

H clips were replaced on half of the plywood decking replaced by the contractor but the remainder of the decking lacked the presence of the H clips. Removal of the roof, felt paper, and decking would be required to meet the requirements for H clips.  At this point, repair by installing 2x4 plates between rafters where no H clips exist is an alternative.
Damaged plywood decking on the northeast side of the residence must be replaced with 1/2” or 5/8” plywood decking to match the existing decking.

The heater vent stack is installed too close to the wood decking. The vent stack should be corrected at the same time the roof jack is replaced.

The repair to the hip rafter is cut diagonally across the rafter and meshed to each piece.  To repair properly, installation of a rafter of the same dimensions should be installed on both sides of the splice covering two thirds of the rafter length.

Conclusion:

Repairs to the home should be accomplished at the owner’s convenience.  The above residence is capable of being restored to its original design by a competent contractor.

.

Sincerely,

Emmett Dammon

La. License 8796
