Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net

# Structural Inspection

August 2, 2012

For: 	Mr. & Mrs. Brown

420 Starling Dr.

Slidell, La. 70461

Ref:    315-317 South Hennessey St.

New Orleans, La.

Construction:

Two-story, wood frame, wood siding veneer and composition shingle roof on a concrete foundation, the home was built circa 1800.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence.  No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. Brown contacted Dammon Engineering to request a structural inspection of the residence for the purpose of purchasing.

Findings:

Upon arriving at the residence it was immediately noted that the front porch of the home is sagging inwards.  It is apparent that rainwater is being allowed to drain into the area where the porch is sagging, then pouring into a hole in the foundation, just behind the center porch column.  This ongoing issue is contributing to settling of the foundation.

During an inspection of the exterior of the ground floor, it was noted that the walls are leaning to the northeast approximately ten inches.  These walls are deteriorating from a multitude of problems, including:  termites, water intrusion and foundation settlement.

During an inspection of the ground floor interior, the foundation was noted to have several cracks and holes allowing additional water intrusion which is further contributing to settlement of the foundation.

The walls supporting the 2nd floor are in very poor condition, resulting from damage from termites, water intrusion and foundation settlement.

The second floor living area was noted to have undergone some degree of renovation.  However, there are several cracks in the sheet rock ceilings and walls indicating settlement has occurred in the foundation of the home, apparently subsequent to the start of the renovations.

The floors of the 2nd floor were also noted to be out of level.

The roof appeared to have been replaced since Hurricane Katrina with architectural shingles.

Mold was noted throughout the residence.

Conclusion:

The residence in its current condition is neither safe nor habitable.  In its current weakened state, the structure is more susceptible to catastrophic damage and/or collapse from even a mild to moderate storm.

To make this a safe and habitable home, it is recommended that a house leveling company raise the 2nd floor of the home.  All of the existing ground floor walls and foundation should be removed. A new foundation with columns supporting the 2nd floor should be constructed.  New exterior walls may then be constructed with an exterior finish of choice.

Sincerely,

Emmett G. (Pete) Dammon, P.E.