DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS					ENGINEERS

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail				   					     985-649-5832

Slidell, LA 70458								            FAX 985-641-5950

# Structural Inspection

August 2, 2012

For: 	Mr. & Mrs. Brown

420 Starling Dr.

Slidell, La. 70461

Ref:    315-317 South Hennessey St.

New Orleans, La.

Construction:

Two-story, wood frame, Wood siding veneer and composition shingle roof on a concrete foundation, the age of the home is approximately 1800 ceria.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Mr. Brown contacted Dammon Engineering to request a structural inspection of the residence for the purpose of purchasing.

Findings:

Upon arriving it was noted that the front porch of the home is sagging inwards. It is apparent that             rainwater is being allowed to drain into the area where the porch is sagging due to a hole in the foundation and causing settling.

After an inspection of the ground floor exterior, it was noted that the walls are leaning to the North East approximately 10 inches. These walls are deteriorating from termites, water intrusion and foundation settlement.

After an inspection of the ground floor interior, the foundation was noted to have several cracks and holes allowing water intrusion therefore causing settling. The walls supporting the 2nd floor are in terrible condition from termites, water intrusion and foundation settlement.

The second floor living area was noted to have renovation started. There are several cracks in the sheet rock ceilings and walls indicating settlement has occurred in the foundation of the home. The floors of the 2nd floor were noted not to be level also.

The roof appeared to have been replaced since Hurricane Katrina with architectural shingles.

Conclusion:

To make this a safe and hittable home, it is recommended that a house leveling company raise the 2nd floor of the home and remove all of the existing ground floor walls and foundation and build a new foundation with columns supporting the 2nd floor with all new necessary exterior walls for the a exterior finish of choice.

Sincerely,

Emmett G. (Pete) Dammon, P.E.