PRELIMINARY COMMERCIAL CHECKLIST

This checklist is merely a preliminary review of the proposed site plan received by our office.  Please understand that our review is limited and shall not constitute approval development or specific plan layout until all components of this checklist are approved by the Planning Department.  Items that are highlighted are items that need to be corrected.
       FILE NAME:  JD Plaza
      APPLICANT:  Dammon Engineering (Chuck Dammon)
           DEVELOPMENT 

                   LOCATION:  345 Gause Boulevard West
                              DATE:  September 10, 2009
                   REVIEWER:  Nancy Durham, CFM
     ZONING DISTRICT:  C-2 Neighborhood Commercial
          PROPOSED USE:  Retail Center
            PREVIOUS USE 

            OF PROPERTY:  Vacant 
GROSS FLOOR AREA:  9,000 sq. ft.
I.
ZONING CONFORMANCE DETERMINATION:  The proposed use of the property is permitted within its zoning district.  (C1=Z.O. Sec. 2.16A03; C2=Z.O. Sec. 2.1601 & C4=Z.O. Sec. 2.1801)

COMMENTS:  O.K. and approved.
II.
SUBDIVISION CONFORMANCE:  Construction across lot lines, except for single-family homes, the subdivision regulations require that no buildings be constructed across lot lines.  (Ord. No. 1679, Subsection 2:219)

COMMENTS:  N/A
III.
CHANGE OF USE DETERMINATION:  When the use of a parcel of property or structure changes, that parcel or structure must comply with all applicable standards established by the Zoning Ordinance.


COMMENTS:  Please see highlighted items in this checklist.
 IV.
NON-CONFORMING USE DETERMINATION:

A.
Structures or uses lawfully permitted by previous Zoning Ordinances, but prohibited by the current ordinance, may not be enlarged or expanded unless the expansion conforms to the provisions of the current ordinance.  (Z.O. Sec. 6.201)



COMMENTS:  N/A
B.
No building or structure or premises where a nonconforming use has ceased for reasons other than those stated (damaged to the extent of not exceeding 50% or its replacement valuation by reason of fire, flood, explosion, earthquake, riot, war, or act of God) for a period of twelve (12) months or has changed to a permitted or conforming use shall again be used as a nonconforming use.  (Z.O. Sec. 6.203-6.204)
COMMENTS:  N/A
V.
REQUIRED YARD AREA SETBACKS:  
A. 
In addition to the setbacks previously identified, the Zoning Ordinance requires 

other applicable setbacks.  The applicable zoning classifications establish front, side, and rear yard setbacks.  ([C1=Z.O. Sec. 2.16A05 & 2.16A06]; [C2=Z.O. Sec. 2.1603 & 2.1604]; [C4=Z.O. Sec. 2.1803 & 2.1804])



Required

Submitted




C2

Front 


25’


122’ 6”
Side


0/3’


R=20’ 4”; L=20’ 4”
Rear


47’ 2”


48’
Height


45’


???
COMMENTS:  Please submit elevations for all sides with height measurement from the ground to the top of the roof.
B. Exceptions to height requirements: 

1.
Compensating Bulk with Open Space – In any district any main structure may be erected or altered to a height in excess of that specified for the district in which the structure is located, provided that each required front, side and rear yard is increased one (1) foot for each one (1) foot of such excess height.  (Z.O. Sec. 10.3(4))  
COMMENTS:  Please submit elevations for all sides with height measurement from the ground to the top of the roof.
2.
Churches, schools, hospitals, sanitariums, public and semi-public; public 
service buildings and institutions.  There shall be no restrictions on the height of such buildings and institutions, except in airport approach and transition zones. There shall be no restrictions on the height of such buildings, provided the front, side, and rear yards required in the district in which such building is to be located shall be increased an additional one (1) foot for each foot of height that the building exceeds the maximum height permitted in such districts.  (Z.O. Sec. 10.3(1))

COMMENTS:  N/A
C.
Height Regulations – Except as otherwise permitted in the Height Overlay Districts, no building or structure for residential or business purposes shall exceed forty-five (45) feet in height above the natural grade of the property at the location of the structure or base flood elevation, whichever is higher.  (Z.O. Sec. 2.1803(A))
COMMENTS:  Please submit elevations for all sides with height measurement from the ground to the top of the roof.
D.
Height Overlay Districts – Height Overlay Districts are hereby created to permit a maximum structure height of seventy-five (75) feet.  If property in the Height Overlay Districts abuts a residential district or use, an additional setback of one (1) foot of setback for every foot in height of the building.  Additional buffering may also be required so as to minimize potential negative impacts to residential areas. [Height Overlay District includes:  East of Airport Road and North of I-12; Lindberg Corridor; North of Gause Blvd. – East of I-10; North of Gause Blvd. West – East of Grand Theater; Northshore Mall and Surrounding Area; South of Gause Blvd. – East of I-10 Including Wal-Mart & Lowes; Summit Development; Target Mall Area; MRO Area; Northshore Business Park – Bella Sera; and Old Spanish Trail – West of I-10]  (Z.O. Sec. 2.1803(B))

COMMENTS:  N/A
VI.
ARCHITECTURAL FEATURES:  
A.
All elevations of all buildings in all commercially zoned districts must have an exterior façade surface material of brick, stone, architectural block, stucco, glass, wood, fiber-cement siding and/or vinyl siding, or any approved combination thereof.  Vinyl siding is permitted for no more than 25% on the total exterior walls.  The architectural surface material must also be included on all building elevations whether facing a street or interior lot line.  Architectural metal panel systems must be approved by the Planning Department.  Standard metal building panels are not permitted.  (Z.O. Sec. 2.2216)

COMMENTS:  Metal Roof / Conventional Stucco Veneer – Please submit details elevations of all sides of the building with all building materials labeled.
B.
All buildings in the Olde Towne Preservation District and Fremaux Avenue Corridor are required to meet the design guidelines and have approval from the review committee.  (Ordinance No. 3486 & Z.O. Sec. 2.2217)
COMMENTS:  N/A
VII.
BUFFER ZONE REVIEW: 

A.
Buffer zone required – The Zoning Ordinance requires that a buffer zone be established between differing land uses as follows:  (Z.O. Sec. 2.2209)







LDR
HDR
C
LI
HI

LOW DENSITY RESIDENTIAL (LDR)

   X
X
X
X

HIGH DENSITY RESIDENTIAL (HDR)


X
X
X

COMMERCIAL (C)






X
X

LIGHT INDUSTRIAL (LI)






X

HEAVY INDUSTRIAL (HI)


X – Buffer Zone Required

       LDR – Single Family Detached Dwelling

       HDR – Attached Dwellings

            COMMENTS:  A buffer zone is required along the rear of the property.
NOTE: IF SUBDIVIDED, BURDEN OF BUFFER FENCE FALLS ON THE COMMERCIAL DEVELOPER. 


B.
Buffer fence – An opaque fence at least six feet in height above the highest point of ground elevation must be located on the property line between the two abutting land uses.  (Z.O. Sec. 2.2210)

1.
The fence must be installed prior to issuance of a building permit.

2.
In situations where fill is placed on a development site, the required fence height shall be computed as follows:






Fence height = (six ft.) + (final grade of site requiring fill – grade of property to be buffered).


COMMENTS:  8’ high opaque buffer fence - O.K. and approved.
C.
Ten foot buffer area established – A ten foot landscaped area is required between the abutting land uses.  Trees must be located in the ten-foot area with a 20-25 foot on center spacing (maximum).  (Z.O. Sec. 2.2211)
COMMENTS:  8-10 trees required along the buffer with 10 trees in the buffer zone.  Please show the buffer zone on the landscape plan that is signed/stamped by a licensed LA Landscape Architect or a licensed LA Landscape Contractor that is contracted to maintain this site. 
VIII.
FENCES (Non-Buffer):  In all commercially zoned districts, fences shall be constructed of wood, iron, vinyl, aluminum, brick, or chain link.  Barbed wire, razor-ribbon, corrugated sheet metal, may not be used.  Chain link may not be used between the building or building setback and a street right-of-way. (Z.O. Sec. 2.2203)  Please state the building material for any fence being added that is not a required buffer fence.


COMMENTS:  N/A
IX.
LAND CLEARING REVIEW:  In order to preserve existing trees on development sites, the Zoning Ordinance requires that a land clearing permit be approved prior to receiving a building permit.  Canopies of protected trees that overhang the property lines of the lot to be developed must be brought to our attention.  The City will give consideration to the following site conditions in its review of land clearing permit applications. (Z.O. Sec. 2.2502)
A. Land Clearing or Protected Tree Removal Permit Required – Application Requirements.  Application for a land-clearing permit shall include the following items:  (Z.O. Sec. 2.2503 (1-2(a-d)))
1.
A plot plan of the proposed development.

2.
A landscape plan (tree survey on proposed site plan), which identifies:

a.
Existing stands of trees on the development site.

b.
The specific location of live oak and magnolia trees twelve (12) inches in diameter and all other trees fifteen (15) inches in diameter as measured twenty-four (24) inches above grade.

c.
Trees or stands of trees proposed for retention on the site.

d.
The intended method for marking reserved trees prior to land clearing shall be identified in the application.  The preferred method of marking is stapling all weather plastic tape to trees.  The use of spray paint for this purpose is specifically prohibited.

NOTE:  The Planning Department has ten (10) days to review this application and conduct a site visit.

COMMENTS:  Please submit land-clearing permit application with tree survey on site plan for review and approval.  If the land is already cleared, a land-clearing application will still need to be submitted stating that the land is already cleared.  

B. Pre-Clearing Inspection Required – The party issued a land-clearing permit shall notify the building official in writing twenty-four (24) hours before starting the land clearing activity. The building official or his designated representative shall inspect the clearing site prior to the start of clearing to assure that the protected trees are identified on site as indicated in the land clearing permit application.  (Z.O. Sec. 2.2505)
COMMENTS:  Please contact the Planning Department for a land clearing

inspection once all the trees are marked for removal.
C. Techniques for Protection of Trees – The following shall be utilized to retain existing trees:  (Z.O. Sec. 2.2506(1-5))
1.
Parking areas and building sites shall be located to preserve existing trees.

2.
Grates or other pervious surfaces shall be utilized within the drip line (outermost limit of horizontal branch extension) of existing trees to allow water and air to reach the tree roots.

3.
Fill shall be prohibited in areas under the drip line of existing trees.

4.
Drastic changes in drainage patterns, which might negatively affect existing trees shall be avoided.

5.
All trees to be retained shall have a perimeter fencing at the extreme outer edge of the tree canopy.  The fencing shall be flagged with yellow caution tape or yellow pennants, and shall remain in place throughout the construction period.  There shall be no activity of any kind inside the perimeter other than hand brush clearing.  No land clearing or building permits shall be issued until the perimeter of all protected trees and tree stands have been properly fenced.

COMMENTS:  Please submit a detailed tree protection plan as outlined above. 

Please schedule a tree site inspection once existing trees are protected.    
D.
Other Notes:

1.
Parcel Size – The ability of a developer to design a site plan that accommodates existing trees is dependent on the size of the parcel in question.  It is much more difficult, for example, to incorporate a specimen tree into a site plan on a 50 ft. by 120 ft. lot than a one acre parcel.

2.
Condition of Existing Trees – Efforts must be undertaken by the developer to minimize the impact of construction activities on trees intended for retention.  However, in all probability, the trees will undergo some stress from related construction.  If the trees are in poor condition prior to start of construction, even minimal additional stress could kill them.  Consequently, such trees are poor candidates for retention.

3.
Growth Conditions – From a road, a densely vegetated parcel appears to be an excellent candidate site for tree retention efforts.  A close examination of such sites often produces a different conclusion.  Trees that have grown along the perimeter of the tree line often are vegetated only on one side.  Trees that have grown in the interior of the site may be only vegetated at the very top of the tree.  This is particularly true of pine trees.  Consequently, in such circumstances, it is not advisable to leave individual trees.  Instead, clusters of trees should be retained.

4.
Types of Trees – Certain species of trees are more desirable than others.  Oaks, magnolias, pecans, hollies, and other hardwoods are more desirable for retention.

5.
Elevation and Drainage Changes – The life system of a tree can be critically affected by placing fill or any impervious surface over its root system.  Likewise, any excavation from the feeder root system to the trunk of the tree will also critically affect it.  However, through proper retention methods such trees can be effectively retained on a given site. (See City’s procedure on “Standards for Tree Retention”)  Amount of fill to be placed on the property will greatly depend on the Flood Elevation Certificate submitted by your Civil Engineer.

COMMENTS:  Please submit land-clearing permit application with tree survey on site plan for review and approval.  If the land is already cleared, a land-clearing application will still need to be submitted stating that the land is already cleared.   

X.
LANDSCAPE REQUIREMENTS REVIEW:  The Zoning Ordinance establishes landscape requirements for commercial developments.

NOTE:  LOUISIANA STATE LAW REQUIRES A LANDSCAPE PLAN MUST BE STAMPED BY A LICENSED LOUISIANA ARCHITECT OR A LICENSED LOUISIANA LANDSCAPE CONTRACTOR.  (Z.O. Sec. 2.2502, La. R. S. 3:3808 (E) & (F), and L.A.C. 7:XXIX.117 (B) & (F))
A.
Perimeter Planting Area – A perimeter planting area ten (10) feet in width must be provided off the public rights-of-way between the rights-of-way and parking area or structures.  Planting materials should achieve a balance between low-lying vertical and horizontal shrubbery and trees. (Z.O. Sec. 2.2513)  Trees shall be placed no more than 20-25 foot on center spacing with a compliment of no less than 10 shrubs per tree.

COMMENTS:  8-10 trees required with 80-100 shrubs.  Please submit a detailed landscape plan from a Licensed LA Landscape Architect or a Licensed LA Landscape Contractor that has a contract to maintain the site showing the required landscaping.
B.
Parking Lot Planting – Parking lot interiors should be designed to provide at least one (1) tree for every twelve (12) parking spaces.  Each tree should be located in a landscaped island of at least two hundred (200) square feet and tree trunks shall be placed at a minimum of five (5) feet from all curbs or wheel guards.  Trees must be distributed uniformly throughout the parking lot so as to provide a canopy effect.  (Z.O. Sec. 2.2514)
COMMENTS:  Please submit a detailed landscape plan from a Licensed LA Landscape Architect or a Licensed LA Landscape Contractor that has a contract to maintain the site showing the required landscaping.
C.
Pedestrian Access Planting – Landscaped areas must be provided between the building faces having access and the parking lot.  Such areas should provide 200 sq. ft. of landscape open space for each 50’ of linear building face.  {Four (4) sq. ft. landscaped area per every linear foot of building face.}  No planting area should be less than five (5) feet wide and planting materials should achieve a balance between low-lying and vertical shrubbery and trees. (Z.O. Sec. 2.2515)
COMMENTS:  Please submit a detailed landscape plan from a Licensed LA Landscape Architect or a Licensed LA Landscape Contractor that has a contract to maintain the site showing the required landscaping.
D. Tree Height – Trees selected shall have a minimum maturity height of at least twenty (20) feet.  (Z.O. Sec. 2.2516)  

COMMENTS:  Please submit detailed legend with minimum maturity height for each type of tree on the landscape plan.
E. Required Inspection – The landscaping will be inspected and approved prior to the issuance of a certificate of occupancy.  (Z.O. Sec. 2.2517)
COMMENTS:  Please schedule a final inspection once all landscaping is complete with the Planning Department.
XI.
PARKING AND ACCESS REVIEW:

A.
Demand Computation – The applicable demand computation for indicated use is calculated as follows:  (Z.O. Sec. 4.201-4.225)



Total Gross Floor Area:  9,000 sq. ft. 



Required Parking Spaces:  30



Handicap Parking Spaces Provided:  2
COMMENTS:  There are 30 parking spaces required for a retail center with all retail (1 @ 300) – O.K. and approved.
B.
Access Analysis:

1. Off-site improvements:  

a. At the location of the proposed development, the anticipated peak 

demand warrants installation of off-site access improvements such as turning lanes or median cuts.  (Ord. No. 2941; COO App E, Part 2)
NOTE:  You must use one of the following qualified traffic study firms for a letter or a study:  Urban Systems, Inc. (N.O 504-523-5511.), Burk-Kleinpeter, Inc. (N.O 504-486-5901.), Neel-Schaffer, Inc. (B.R.225-924-0235), Evans-Graves Engineering (Met. 504-836-8190), Sain Associates, Inc. (Birm., AL 205-940-6420), Krebs, LaSalle, LeMiex Consultants, Inc. (Met. 504-837-9470), Kelly J. McHugh & Associates, Inc. (Mandeville 985-626-5611), Richard C. Lambert Consultants (Mandeville 985-727-4440), Dean Tekell Consulting (Laf. LA 337-988-5211) or J.V. Burkes & Associates, Inc. (Slidell 985-649-0075).  

COMMENTS:  Please submit a traffic impact study (2 copies) or letter stating one is not required.
b.
All new or expanding developments, typically generating 100 hourly trips in the peak direction on state highways and on local public or private streets, with an access point within 0.25 of a mile of a state highway is required to have the traffic study approved by DOTD.  (Title 70, Part I, Chapter 11, §1101)
COMMENTS:  Please submit Letter of Compliance from DOTD approving the traffic impact study only if the TIA is 100 or more trips.
c.
If a driveway attaches to a State road, a copy of the LA DOTD driveway permit will be required to be submitted with package. (Contact LA DOTD at 985-375-0130).  Some of the State roads within Slidell City limits are:  Gause Boulevard, Gause Boulevard West (Highway 190), Gause Boulevard East, Fremaux Avenue (US Highway 190), Shortcut Road (US Highway 190), Robert Boulevard (County Highway 1091), Front Street (US Highway 11), Pontchartrain Drive (US Highway 11), US Highway 11, Old Spanish Trail (LA Highway 433), and Bayou Liberty Road (LA Highway 433).  Contact LA DOTD for a current list.

COMMENTS:  Please submit a copy of the LA DOTD driveway permit.
 
2.
Access point location:

a.
Access points should not be located so as to encourage difficult or hazardous vehicular movements on adjacent streets. 
COMMENTS:  O.K. and approved.
b.
Unless dictated by the size or configuration of a site, the centerline of access driveways should not be located closer than seventy-five (75) feet from the centerline of access driveways on or off of the site, or, in the case of a corner parcel, closer than seventy-five (75) feet to an adjacent right-of-way line.  (LA DOTD 6.7.1(6))
COMMENTS:  Please show the locations of the driveways that are adjacent to and across from this property.
c.
Off-street parking regulations in the C-1 district are the same as the C-2 zoning district (Z.O. Sec. 2.1605).  If a lot or parcel is contiguous to a residentially zoned parcel, no off-street parking spaces or driveways shall be constructed within twenty-five (25) feet of the rear or side lot line which is contiguous to the residential property. (Z.O. Sec. 2.16A08)
COMMENTS:   N/A
d.
Corner parcels which front a thoroughfare on one side and a residential street on the other side should not locate access points on the residential street.  
COMMENTS:  N/A
e.
Whenever possible, access driveways should be aligned directly with access driveways on the opposite side of the street.  This requirement is not applicable on streets divided by a median.  (LA DOTD 6.7.1(3))
COMMENTS:  Please show the locations of the driveways that are adjacent to and across from this property.
f.
Access driveways should be aligned with median cuts or, in cases where streets dead-end at a development site, with the street.  (LA DOTD 6.7.1(3))
COMMENTS:  N/A
3.
Number of access points – The site plan should include the minimal number of access points necessary to safely and efficiently service the site.

COMMENTS:  1 – O.K. and approved.

4.
Design of access driveways:

a.
Driveway width-commercial driveways should have a minimum width of twelve (12) feet for one-way traffic and twenty-four (24) feet for two-way traffic with a maximum width of thirty-five (35) feet.  (LA DOTD 6.7.4(3))
COMMENTS:  24’ – O.K. and approved.
b.
Turning radius – Commercial driveways require a minimum turning radius of at least fifteen (15) feet.

COMMENTS:  Follow DOTD regulations.
c.
Access curb – Depressed or ramp type entrances are required on all commercial driveways.  Typical 1 inch raised lip per LA D.O.T.D. permit requirements.

COMMENTS:  Get DOTD permit if required.
d.
Consistency with state requirements – All entrances, turning lanes and other improvements located on state highways require approval of the Louisiana Department of Transportation and Development.  The state’s standards may be more stringent than the City’s standards.  Consequently, it is very important that related state permits be applied for as soon as possible after City approval.  Failure to receive state approval for such improvements may delay occupancy of the development.

COMMENTS:  Get DOTD permit if required.

5.
Internal traffic lanes:

a.
Travel lane width – Travel lanes servicing 30, 45, 60 and 90 degree adjacent parking should provide travel lane widths as follows:



30 DEGREE PARKING – 11-13 FEET



45 DEGREE PARKING – 13-15 FEET



60 DEGREE PARKING – 18-20 FEET



90 DEGREE PARKING – 24-26 FEET

COMMENTS:  24’, 30’ & 24’ – O.K. and approved.
b.
Dead end travel lanes – Dead end travel lanes should only be provided when servicing 90 degree adjacent parking.  In such circumstances, an area at least 10 feet deep by 24 feet wide backing provided at the end of the lane to accommodate turning movements.

COMMENTS:  Both = 10’ X 24’ – O.K. and approved.
c.
General design considerations – Travel lanes should be located to maximize efficient traffic flow and minimize difficult turning maneuvers.  Critical access lanes may require an expanded turning radius to accommodate fire protection vehicles.

COMMENTS:  Please check with fire prevention for the radius.
d.
Parking lots shall not be designed to require vehicles to back out onto public rights-of-way or major thoroughfares to exit the site.

COMMENTS:  O.K. and approved.
6.
Parking stall size – One automobile parking space shall be an area not less than nine (9) feet by twenty (20) feet.  This stall size must be appropriately adjusted for other angular configurations.  Provided that a lot is double striped to accommodate targeted parking, an 8.5’stall is allowed for C-6 Zoning Districts.  (Z.O. Part 9 Definitions – 9.24)
COMMENTS:  9’ X 20’ – O.K. and approved.
7.
Handicap parking spaces and signage are required to meet the ADA regulations.  These regulations can be found at the ADA website:  http://www.ada.gov/ under the ADA Design Standards (http://www.ada.gov/stdspdf.htm). 

COMMENTS:  Please follow the ADA regulations.
8.
Parking area surface – Parking areas should be surfaced with at least four (4) inches of concrete, four (4) inches of bituminous asphalt paving, or a pervious material such as grass crete.  Shell, gravel, or limestone is not an acceptable surface material. (Z.O. Sec. 4.1 & 4.2)
COMMENTS:  Please submit parking area surface detail or statement of pavement.
C.
Off-Street Loading Requirements:

1.  
Every hospital, institution, hotel, commercial or industrial building or use having a gross floor area in excess of 7,500 square feet of space and requiring the receipt of distribution by vehicle of material and merchandise shall have at least one permanently maintained off-street loading space 10 feet X 45 feet for each 7,500 square feet of gross floor area or fraction thereof and so located as not to hinder the free movement of pedestrian and vehicles over sidewalks, streets, and alleys.  (Z.O. Sec. 4.301)

COMMENTS:  O.K. and approved.


2.
Retail operations, wholesale operations, and industrial operations with a gross floor area of less than 7,500 square feet shall provide sufficient off-street loading space (not necessarily full berth if shared by an adjacent establishment) so as not to hinder the free movement of pedestrians and vehicles over a sidewalk, street, or alley.  (Z.O. Sec. 4.302)
COMMENTS:  N/A
3.
Loading zone requirements in the C-1 district are the same as the C-2 zoning district (Z.O. Sec. 2.1606).  If a lot or parcel is contiguous to a residentially zoned parcel, no loading areas shall be constructed within twenty-five (25) feet of the rear or side lot line which is contiguous to the residential property.  (Z.O. Sec. 2.16A09)
COMMENTS:  N/A
XII.
PEDESTRIAN ACCESS:  Sidewalks shall be provided within the public right-of-way adjacent to new residential and commercial development as follows (COO App. B, Sec. 4.2 & COO App. B, Sec. 4.501):
Development Types

Minimum Width

Minimum Thickness

Single Family Detached

4 ft.



4 in.
Attached Residential


5 ft.



4 in.

Commercial



10 ft.



4 in.

Such sidewalks generally should be located one (1) foot from the property line within the right-of-way.  However, when this location would cause the loss of specimen trees, the sidewalk should be designed to maintain as many trees as possible.  NOTE:  Sidewalks are not required along LA D.O.T.D. thoroughfares per the State of Louisiana.  Sidewalks are not required in residential subdivisions where the majority of the lots have previously been built upon without public sidewalks and where there are no public sidewalks within 200 feet of the lot or lots being improved on the same side of the street.

COMMENTS:  N/A
XIII.
REQUIRED YARD AREA SETBACKS:  In addition to the setbacks previously identified, the Zoning Ordinance requires other applicable setbacks.

A.
Commercial Development – Whenever a commercial development abuts the rear of a residential development, the rear yard setback for the residential zoning district applies as the rear yard setback for the commercial property.  (Z.O. Sec. 2.1604(b) & 2.1804(b))
COMMENTS:  48’ – O.K. and approved.
B.
Accessory Uses – The Zoning Ordinance regulates the location of accessory buildings and uses.  (Z.O. Sec. 10.4)
COMMENTS:  N/A
XIV.
LOCATION OF DUMPSTERS:  

A.  
Section 23-13 of the City of Slidell Solid Waste Ordinance requires every commercial site to provide containers capable of handling a week’s accumulation of trash in a sanitary manner.  The location of commercial dumpsters shall be located on the site plan and should be incorporated into the design for ease of access.  The dumpster shall be screened with a solid wood fence and gate or other suitable materials approved by the Planning Department. (COO Sec. 23-13)
COMMENTS:  O.K. and approved.
B. Section 23-16 of the City of Slidell Solid Waste Ordinance states that the location of the construction dumpsters shall not be located within 10 feet of an adjacent dwelling, blocks the sidewalk, or placed on any public street or public alley.  (COO Sec. 23-16)
COMMENTS:  O.K. and approved.
XV.
LOCATION OF LITTER RECEPTACLES:  The receptacles shall be located on the site plan and should be incorporated into the design for ease of access.

A.
The state’s litter law requires every property which is held out to the public as a place for parking consisting of fifteen or more parking spaces shall be responsible for the procurement, placement, and maintenance of litter receptacles. (LA RS 30: §2535(A))
COMMENTS:  3 – O.K. and approved.
B.
Litter receptacles shall be required at any parking lot consisting of thirty or more parking spaces operated for public use.  (LA RS 30: §2535(C))

COMMENTS:  N/A
XVI.
LOCATION OF FIRE HYDRANTS:  Location of fire hydrants shall be indicated on the site plan.  Water mains shall be designed of sufficient size to provide at least one (1) fire hydrant to within three hundred (300) feet of all points of a structure per “hose lay”, within a commercial and industrial area and comply with all rules and standards of the state rating bureau. (Subdivision Regulations: Sec. 4.702 (2))   Structures requiring sprinkler systems shall have a hydrant "dedicated" to the sprinkler system. An additional hydrant shall be required. The hydrants shall not be fed by the same service line/main, when possible. Location and connection of the hydrant units shall be subject to the approval of the fire prevention bureau, the building official, and city engineer. (Subdivision Regulations: Sec. 4.702 (3))

COMMENTS:  The Planning Department no longer reviews the location of fire hydrants.  Please check with the Engineering Department and Fire Prevention for approval.

XVII.
SIGNS:  All types of signage require permits.  Experience has indicated that in all too many cases, signage is an afterthought.  Signage should be part of the design process and considered an integral part of the site planning process.  A sign review and approval will be made by the Planning Department before a permit is issued.  Submit the sign packet as a separate packet for the types of signage listed below for review and approval. Reminder: Every sign will require a building permit.  (Z.O. Sec. 503 & 504)

Freestanding Signs (Single or Complex)


Fascia Signs (Building or Wall Signs)


Directional Signs


Interstate Signs (if applicable)


Banners (Temporary Signs)


COMMENTS:  Please submit a sign permit application for any new freestanding or fascia signs.

XVIII.     EXTERIOR LIGHTING:  Section 2.19(J) in the Zoning Ordinance regulates glare.       
A. Commercial and Industrial Light Standards – No use in any district shall be operated so as to produce direct sky-reflected glare, or direct illumination, across the adjacent property line from a visible source of illumination of such intensity as to create a nuisance, or traffic hazard, or detract from the use or employment of adjacent property.  Residential lighting fixtures shall be directed or shaded to prohibit the intensity of light to exceed one-half foot-candle as measured at any adjacent residential property line.  Commercial lighting fixtures shall be directed and shaded to prohibit the intensity of light to exceed one (1) foot-candle as measured at any adjacent property line.  (Z.O. Sec. 2.19 (J(4)))
NOTE:  Exterior lights are required to be recessed in the canopy, no bubble lights allowed for gas stations.

COMMENTS:  Please submit exterior lighting details that show number of foot-

candles measured at property lines, and/or submit a letter stating that Section 2.19(J) of the City of Slidell Zoning Ordinance will not be violated.
B.
Hours of Operation – Lighting of parking or loading areas shall, except for emergency or safety lighting, cease at or before the hour of midnight when these areas face a residential district where the distance from the nearest light to the nearest residence is less than one thousand (1,000) feet, except in those instances in which a business establishment is open and business is being conducted after midnight.  (Z.O. Sec. 2.19 (J(4(a)))
COMMENTS:  Please submit letter from business owner that states the business hours and that this requirement is understood and would be followed.    

C.
Blinking or Intermittent Lights – No exterior lights that blink or shine with an intermittent phase are permitted in any district, except as part of Christmas decorations.  (Z.O. Sec. 2.19 (J(5))  This includes signs.

COMMENTS:  Please submit letter from the business owner that this requirement will be met. 

XIX.
NOISE:  
A.
Maximum Permissible Sound Levels by Receiving Land Use – No person shall operate or cause to be operated on private property, any source of sound in such a manner as to create a sound level which exceeds the limits as established in the following table in accordance with the time of day and zoning applicable to the receiving land use category as follows (C.O.O. Sec. 13-1 (c (1))):

Receiving Land Use Category
Time


Sound Level Limit (dBA)

Industrial



At all times

85 dBA


Commercial



7:00 am to 10:00 pm
75 dBA







10:00 pm to 7:00 am
65 dBA


Multi-Family



7:00 am to 10:00 pm
60 dBA







10:00 pm to 7:00 am
50 dBA


Motor Vehicles


At all times

85 dBA


Schools (within 20’)


7:00 am to 4:00 pm
50 dBA


COMMENTS:  Please make sure you abide by these during construction.
B.
Permissible Time for Construction Activity, Operation of Domestic Tools (C.O.O. Sec. 13-1 (c (8 (a-b))))
1.
It shall be unlawful for any person to do, perform or engage in any construction work of any nature between the hours of dusk to daw, or at an time on Sunday, if any such activity shall cause noised whose levels result in excess of 40 dBA at the receiving property line.

2.
It shall be unlawful to operate or permit the operation of any mechanically powered saw, drill, sander, grinder, lawn or garden tool, snow blower or similar device used outdoors in residential areas between dusk to dawn, so as to cause the sound level limits established in this section to be exceeded at the property line.  At no time shall the sound level caused by or emitted from any of the above tools exceed 80 dBA at the property line.


COMMENTS:   Please make sure you abide by these during construction.
PAGE  
1

