[image: image1.jpg]IDAMMON

ENGINEERING | NLC.

J  Archiltectls & Engineers




554 Old Spanish Trail
Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net


Site Description

The project is located at 200 West Gloria Switch Road, Carencro Louisiana. The property is bounded by Evangeline Throughway (Hwy 49) on the East Side, Four Park road on the West Side and West Gloria Switch Road on the South Side.
The overall lot size of lot 4 is 1.894 Acres. The site will be the new home of a 7000 sq. ft. Mexican restaurant named El Paso.
Current Drainage

The overall drainage ditch in this area is from the north along Hwy 49 to the south between our adjoining properties and under West Gloria Switch Road to the south.

The subject property currently drains from the east (existing concrete parking lot) onto the subject property into a swale and continues west into a 12” CCP and into the existing ditch at the rear of the property. 
Study Methodology

The modified rational method was used to determine peak flows for the 10 year storm event for the drainage and retention.

The rainfall distribution was derived from the 2011 Louisiana Department of Transportation and Development hydraulic Manual.
Proposed drainage

The existing street in front of the property currently sheet flows to the west and will be captured in new catch basins in the new driveway entrances. The roof will drain into subsurface drainage and into the catch basins. The parking lot will drain into these catch basins and be retained until discharged in the existing rear drainage ditch. The outflow device is 12” PVC pipe-30 linear feet with a 9” orifice at a slope of 1.4%.  
Bill Kingsmill’s contact information is as follows:
William Kingsmill (kingsmillprop@bellsouth.net)
Cell: (504) 528 3838
1200 Business Highway 190, Suite 13
Covington, LA 70433
 
Lawrence Wm. Haik, Sr
Republic Commercial Properties, LLC                
1539 Royal Palm Dr.
Slidell, LA 70458
(985) 960-7033
 
