February 24, 2000

Allied Mortgage

1300 Gause Blvd

Slidell, LA  70458

Attention:  Jamie

Foundation Inspection, Doublewide Mobile Home

Address:
107 Racoon Run,



Pearl River, LA

Owner:
Walter Adams

Phone:
863-5073

The foundation and doublewide mobile home are both new, and the foundation was installed by the company that set the unit up.

  I found the mobile home to have approximately sixty-five (65) concrete block piers.  They were installed at regular intervals along each side of the unit, and along the interior-mating surface of the doublewide. Additional piers were installed at door openings.

  Four (4) inch thick concrete pads were used under each of the piers, as spread footing.   These units were 18”x18” along the exterior and 24”x24” under the interior wall.  Hurricane tie downs were installed as required to meet code.

  I found the foundations system for the unit to be in very good condition.  It is my professional opinion that the foundation system of this doublewide meets code and is adequate for the doublewide mobile home.

Sincerely,

Emmett G (Pete) Dammon P.E.,

EGD/pc   

