December 6, 2001

Bobby Hurley

Brice Building Company, Inc.

3516 Hessmer Avenue

Metairie, LA  70010

RE:
Camellia Gardens

Dear Bobby,

Answer to RFI No.6

Expansion Joints:

Slab: Slab shall have two expansion joints per breezeway: install ½”x4” expansion joint.

Use smooth dowels at 16” O.C. 12” each way fully lubricated with Vaseline.

See attached Expansion Joint Site Plan.

Brick Veneer: butt brick from breezeway to bldg. do not attach. Install silicone connector with Styrofoam backer rod from top to bottom.

The sheet rock walls, sheet rock ceiling and composition roof will not receive expansion joints.

Additional Information:

1) Install an exhaust wall fan in bldg. #1 Mechanical room. Chuck Sardi with Pontchartrain Mechanical will provide this information.

2) Bldg.#1 First floor, Sheet A-2: The door mark leading into the Receptionist/Secretary Area shall change from 9 to 18. 18 are identified as Solid Core door in hollow metal frame with Fire stat controlled holders, 1 Hr rated w/panic hardware and with auto closures. See attached DWG.

3) Tenant Fire Separation Fire Protection: Typical Roof/Ceiling Assembly One Hour Rating,

2 layers of 5/8” type “X” gyp. Bd. attached to the bottom chord of roof trusses and roof framing. System shall comply with gypsum association, Fire Test: FMFC 172, 2-25-72; GA. File No. RC2601.

See Attached cut sheet.

4).  Bldg.#3 Foundation Plan: A grade beam and one cable shall be added to support the load of the roof for  the offsets in the hallway. Please see sheet S-4 for the change.

