ADDENDUM #1

August 2, 2000

To:

All Plan Holder

Project:
Margon Court Culverts



City of Slidell, LA



Slidell Project # 100-80

Please be advised that the following pipe will be acceptable for the project.

1. HANCOR 15” F477 pipe meeting AASHTO252 Type, 245 Type and MPT-97.

Joining system shall be bell and spigot, with rubber gaskets.

2. TEE inlets for existing drainpipes will be permitted.

3. SDR 26 sewer pipe will also be acceptable.

The above substitutions will require that:

1. The contractor must start laying pipe at the far end, away from his supply point, and work backwards to avoid running heavy equipment over freshly laid and compacted areas of the project.

2. Strictly adhere to the bedding and back filling details shown on drawings.

3. All joints are to be wrapped with fabric.

Sincerely,

Emmett G (Pete) Dammon P.E.,

EGD/plc

