March 20, 2002

Mr. Danny Brown

McMath Construction

215 St. Ann Drive, Ste. 3

Mandeville, Louisiana  70471

RE:
S.T.A.R.C./New Daycare Facility

Dear Mr. Brown,

Please provide STARC with a price to add the following beams to the portico.  During installation, it became obvious that it should be strengthened to eliminate possible flexing of the roof during high winds.

Also, add item #3 below, as it somehow did not print on the final drawing.


Add the following members:

1. Add two 4” round pipe columns in wall of building to support 4” x 14” truss beam above wall.  Add these at locations shown on drawing.

2. Add one each 4” x 8” x ¼” rectangular tube at 1/3 points of portico roof (two places) position to connect inside 14” main beam section and below the 6” roof purlins.  Run from crown of roof to eave.  Bevel bottom at outside purlin.

3. Add one 4” x 14” x ¼” member across front to form bottom chord of truss.  Add one 4” x 14” x ¼” vertical member and one 45  angle member at each side of vertical, see attached drawing.

I appreciate your help on this matter.  If you have any questions, or require any further information, please feel free to call.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Attachment:
Drawing

cc:
Steve Duvernay/S.T.A.R.C.


Mary Morgan/S.T.A.R.C.
