Twin Oaks

Hattiesburg, Mississippi

Date of Inspection:
February 18, 2004

Twin Oaks

Bldg. #9

144 Units/12 Apartments

1) All wood in these buildings shall be sprayed for mold

2) Remove water from floors

3) Fire caulk all wiring and plumbing penetrating top/bottom plates as well as fire barriers

4) Change front doors to handicap

5) Install hat channel on ceilings, install sheet rock on ceilings and walls

6) Connect A/C units

7) Install fire dampers in bathrooms

8) Connect all power to panels and coat ends with oxidizer compound

9) Install insulation in walls and third floor ceilings

10) Properly install straps on all vent and sewer stacks

11) Complete all stairs and handrails

12) Seal around second and third floor tub drains

13) Install siding

14) Complete all porches

15) Complete roof over stairs

16) Verify can lights on third floor kitchen are fire rated

17) Patch all holes in sheetrock for firewalls and separations

18) Install smoke detectors according to code

___________________________
Date:
____________________


Chuck Dammon

         VP/Project Manager
